1. Combined Nodes Table

Structure

The combined nodes table integrates all nodes (stringers, ribs, spars) into a unified format with special tags to handle specific cases like spars and root nodes.

| Column | Description | Notes |
| --- | --- | --- |
| 1 | local_id | Unique identifier for each node. |
| 2 | x | X-coordinate of the node. |
| 3 | y | Y-coordinate of the node. |
| 4 | rib_index | Rib index for the node. |
| 5 | stringer_index | Stringer index for the node. |
| 6 | tag | Node tag (e.g., "front spars", "rear spars", "stringer"). |

Special Index Values

- rib_index:

  - -1: Root ribs.

  - -2: Front spar ribs.

- stringer_index:

  - -1: Front spar stringer.

  - -2: Rear spar stringer.

Purpose

- Centralizes all node data in a unified format for easy access.

- Tags ("front spars", "rear spars", etc.) enable filtering for specific applications.

Example

| local_id | x | y | rib_index | stringer_index | tag |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.0 | 0.0 | 1 | 1 | "stringer" |
| 2 | 0.1 | 0.1 | 2 | 1 | "stringer" |
| 3 | 0.0 | 0.0 | -1 | -1 | "front spar" |

2. Quadrilateral Surfaces Table

Structure

Each row represents a quadrilateral surface, with detailed metadata.

| Column | Description | Notes |
| --- | --- | --- |
| 1 | local_id | Unique identifier for each surface. |
| 2 | node_1 | Node ID for the first corner of the quadrilateral. |
| 3 | node_2 | Node ID for the second corner of the quadrilateral. |
| 4 | node_3 | Node ID for the third corner of the quadrilateral. |
| 5 | node_4 | Node ID for the fourth corner of the quadrilateral. |
| 6 | stringer_1 | Stringer index corresponding to node_1. |
| 7 | stringer_2 | Stringer index corresponding to node_2. |
| 8 | rib_1 | Rib index corresponding to node_1. |
| 9 | rib_2 | Rib index corresponding to node_2. |
| 10 | tags | Surface tag (e.g., "quad regular", "quad irregular"). |
| 11 | area | Calculated area of the quadrilateral. |
| 12 | aspect_ratio | Aspect ratio of the quadrilateral (max/min diagonal). |

Purpose

- Provides comprehensive metadata for quadrilateral surfaces.

- Facilitates filtering by tags (e.g., "quad regular", "quad irregular").

- Supports validation with geometric metrics (area, aspect_ratio).

Example

| local_id | node_1 | node_2 | node_3 | node_4 | stringer_1 | stringer_2 | rib_1 | rib_2 | tags | area | aspect_ratio |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 101 | 102 | 201 | 202 | 1 | 2 | 1 | 2 | "quad regular" | 0.5 | 1.2 |

3. Triangular Surfaces Table

Structure

Each row represents a triangular surface, with detailed metadata.

| Column | Description | Notes |
| --- | --- | --- |
| 1 | local_id | Unique identifier for each triangle. |
| 2 | node_1 | Node ID for the first vertex of the triangle. |
| 3 | node_2 | Node ID for the second vertex of the triangle. |
| 4 | node_3 | Node ID for the third vertex of the triangle. |
| 5 | P1 stringer | Stringer index corresponding to node_1. |
| 6 | P1 ribs | Rib index corresponding to node_1. |
| 7 | P2 stringer | Stringer index corresponding to node_2. |
| 8 | P2 ribs | Rib index corresponding to node_2. |
| 9 | P3 stringer | Stringer index corresponding to node_3. |
| 10 | P3 ribs | Rib index corresponding to node_3. |
| 11 | tags | Surface tag (e.g., "tri root", "tri irregular"). |
| 12 | area | Calculated area of the triangle. |
| 13 | aspect_ratio | Aspect ratio of the triangle (longest edge / shortest edge). |

Purpose

- Extends metadata to triangular surfaces for root areas and irregular zones.

- Facilitates filtering by tags ("tri root", "tri irregular").

- Supports validation with geometric metrics (area, aspect_ratio).

Example

| local_id | node_1 | node_2 | node_3 | P1 stringer | P1 ribs | P2 stringer | P2 ribs | P3 stringer | P3 ribs | tags | area | aspect_ratio |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 101 | 102 | 201 | 1 | 1 | 2 | 2 | -1 | -2 | "tri root" | 0.3 | 1.5 |

Implementation Notes

- Combined Nodes Updates:

  - Ensure special indices (-1, -2) are included and appropriately tagged.

- Quadrilateral and Triangular Surfaces:

  - Use functions to dynamically populate metadata and calculate area and aspect_ratio.

- Tags:

  - Tags like "quad regular", "quad irregular", "tri root", and "tri irregular" should be assigned based on surface type and location.

- Validation:

  - Use area and aspect_ratio for geometric quality checks.

Future Enhancements

- Connectivity Metadata:

  - Add columns for adjacent surfaces (e.g., adjacent_surface_1, adjacent_surface_2) for mesh continuity.

- Material and Element Properties:

  - Include columns like material_id and element_type for FEA-ready surfaces.

- Boundary Conditions:

  - Add metadata for boundary edges (e.g., bc_edge_1, bc_edge_2) for FEA constraints.