Documentation for Rib Surface Construction

This documentation provides a clear and standardized framework for constructing the rib surfaces in the FEM model. It ensures consistency in defining elements and facilitates accurate communication and interpretation throughout the thesis.

Purpose

- Thesis Reference Guide: To ensure consistency and clarity in defining rib surfaces, making the methodology reproducible and understandable in the thesis.

- Effective Communication: To standardize the process of rib surface creation and its integration with other structural elements (e.g., rods), ensuring alignment and accurate understanding between collaborators.

Rib Surface Construction

General Description

- Type of Element: Rib surfaces are modeled as shear panels (SHEAR).

- Associated Elements: The rib surface is bounded by:

  - Cordones de costilla (RODs): Horizontal rods connecting the stringers along the extrados and intrados.

  - Montantes de costilla (RODs): Vertical rods connecting the extrados and intrados of the ribs.

  - Montantes de larguero (RODs): Vertical rods connecting the rib surfaces to the spars.

Surface Definition Order

The rib surface follows a counter-clockwise (CCW) order starting from the top-left corner when viewed from the tip to the root of the wing. This ensures consistency with the rest of the structure.

Node Order for Rib Surface

For a rib with four bounding nodes, the order of nodes is as follows:

- Top-Left: Starting point on the extrados at the leading edge.

- Bottom-Left: Moving downward along the leading edge to the intrados.

- Bottom-Right: Traversing along the intrados to the trailing edge.

- Top-Right: Moving upward along the trailing edge to the extrados.

Steps for Rib Surface Construction

- Identify Nodes:

  - Extract the relevant nodes for the rib from the combined_nodes_3D table.

  - Ensure that the nodes correspond to the rib’s extrados and intrados at the leading and trailing edges.

- Follow the Counter-Clockwise Order:

  - Use the top-left node as the starting point.

  - Traverse the nodes in the following sequence:

    - P1P_1P1​: Top-left (extrados, leading edge).

    - P2P_2P2​: Bottom-left (intrados, leading edge).

    - P3P_3P3​: Bottom-right (intrados, trailing edge).

    - P4P_4P4​: Top-right (extrados, trailing edge).

- Assign Element Properties:

  - Tag the surface as rib shear.

  - Calculate geometric properties like area and aspect ratio, if required.

- Boundary Rods:

  - Define Cordones de costilla and Montantes de costilla separately as rod (1D) elements along the edges of the rib.

Key Considerations

- Geometric Alignment:

  - Ensure the top-left node is always on the extrados at the leading edge, maintaining consistency in node selection and order.

  - Verify alignment of the rib surface nodes with the spars and skin.

- Mesh Compatibility:

  - Maintaining a CCW order ensures compatibility with downstream FEM tools like Patran/Nastran.

- Integration with Rods:

  - The rods along the edges must align perfectly with the shear surface to avoid gaps or misalignments in the model.

Example

Given Rib Nodes:

| Node ID | x | y | z | Location |
| --- | --- | --- | --- | --- |
| 1 | 0.0 | 0.5 | 0.1 | Top-left (extrados) |
| 2 | 0.0 | 0.0 | -0.1 | Bottom-left (intrados) |
| 3 | 1.0 | 0.0 | -0.1 | Bottom-right (intrados) |
| 4 | 1.0 | 0.5 | 0.1 | Top-right (extrados) |

Node Order for Surface:

- P1=P_1 =P1​= Node 1 (Top-Left).

- P2=P_2 =P2​= Node 2 (Bottom-Left).

- P3=P_3 =P3​= Node 3 (Bottom-Right).

- P4=P_4 =P4​= Node 4 (Top-Right).

Final Notes

- Following the CCW order starting from the top-left ensures compatibility with the global node ordering conventions.

- The ribs are modeled as single shear surfaces, bounded by rods for structural stability.

- This framework ensures a reproducible and accurate definition of rib surfaces for your FEM model.