tfg_guia_Modelo FEM de la estructua de un ala.pdf

PDF

Etapa-3-predimensionado-cajon-ala-metalico-compuesto.pdf

PDF

We have some problems. The forces in the calculated does not have a designated coordinates. These forces must be applied to the simulation, later it must have a designated node to properly simulate. Other problem is that from stage 2 to stage 3, we do not have a proper structure on how to read this forces. We concluded that in stage 3, there are two parts, the first part is to have an idea on the dimensions. The second part is the analysis for the boxes/cajón. The problem is that the geometry of the wing is variable box. This is where the coordinates of the forces also intervenes, not only in stage 5 simulation. For every point ribs, we need to do some form of analysis. I will attach the necesary information, so you can follow me. This is the function for calculating final forces: "function results = calculate_final_forces(ala, avion, cargas) % CALCULATE_FINAL_FORCES Computes the final forces applied to the wing % considering aerodynamic and mass forces, following the TFG guide. % % Inputs: % - ala: Struct containing wing geometry and rib locations. % - avion: Struct with aircraft properties (MTOW, n, etc.). % - cargas: Struct with aerodynamic load distribution. % % Outputs: % - results: Struct containing final aerodynamic and mass forces. %% 📌 Load Inputs MTOW = avion.MTOW; n = avion.datosEstructural.n; Lw = avion.geometria.Lw; numero_costillas = ala.numero_costillas; pendiente_perpendicular_larguero_posterior = ala.geometria.pendiente_perpendicular_larguero_posterior; x_local_ala = avion.coordenadas.x_local_ala; x_l = ala.x_l; y_l = ala.y_l; coord_aerodinamica_costillas_punto_medio = ala.coord_aerodinamica_costillas_punto_medio; % coord_interseccion_paralela_costillas_pasa_por_A = ala.coord_interseccion_paralela_costillas_pasa_por_A; schrenk = cargas.schrenk; % ✅ Schrenk’s Interpolation l = spline(x_local_ala, schrenk, coord_aerodinamica_costillas_punto_medio); l = l * n * MTOW * 2 / Lw^2; % Scale by load factor and weight %% ✨ **Discretization of Aerodynamic Forces** size_L = length(l) - 2; L = zeros(size_L, 1); x_L = coord_aerodinamica_costillas_punto_medio(2:end-1); y_L = zeros(size(x_L)); for i = 2:numel(x_L) - 1 L(i-1) = (1/6) * (x_L(i+1) - x_L(i)) * (2*l(i) + l(i+1)) ... + (1/6) * (x_L(i) - x_L(i-1)) * (l(i) + 2*l(i-1)); end y_intercept = y_l - pendiente_perpendicular_larguero_posterior * x_l; coord_interseccion_paralela_costillas_pasa_por_A = zeros(size(y_intercept,1),2,3); % dimension es num_costillas y (X,Y) y (larguero anterior;larguero posterior,eje estructural) %% ✅ **Compute Weight Forces** peso_ala = 0.1 * MTOW; % Estimated from guide peso_combustible = 0.2 * MTOW; W_ala = peso_ala * (x_L / max(x_L)).^3; % Distributed by chord^3 W_comb = peso_combustible * (x_L / max(x_L)).^3; %% 🔹 **Discretization of Forces at Rib Intersections** h_A_aero = zeros(size_L, 1); h_R_aero = zeros(size_L, 1); l1_A = zeros(size_L, 1); l2_A = zeros(size_L, 1); l1_R = zeros(size_L, 1); l2_R = zeros(size_L, 1); P_A_aero = zeros(size_L, 1); P_R_aero = zeros(size_L, 1); R1_A_aero = zeros(size_L, 1); R2_A_aero = zeros(size_L, 1); R1_R_aero = zeros(size_L, 1); R2_R_aero = zeros(size_L, 1); for i = 1:size_L h_A_aero(i) = norm([x_L(i) y_L(i)] - coord_interseccion_paralela_costillas_pasa_por_A(i+1, :, 1)); h_R_aero(i) = norm([x_L(i) y_L(i)] - coord_interseccion_paralela_costillas_pasa_por_A(i+1, :, 2)); l1_A(i) = norm(ala.costillas(i+1, :, end) - coord_interseccion_paralela_costillas_pasa_por_A(i+1, :, 1)); l2_A(i) = norm(ala.costillas(i+2, :, end) - coord_interseccion_paralela_costillas_pasa_por_A(i+1, :, 1)); l1_R(i) = norm(ala.costillas(i+1, :, 1) - coord_interseccion_paralela_costillas_pasa_por_A(i+1, :, 2)); l2_R(i) = norm(ala.costillas(i+2, :, 1) - coord_interseccion_paralela_costillas_pasa_por_A(i+1, :, 2)); P_A_aero(i) = L(i) * h_R_aero(i) / (h_A_aero(i) + h_R_aero(i)); P_R_aero(i) = L(i) * h_A_aero(i) / (h_A_aero(i) + h_R_aero(i)); R1_A_aero(i) = P_A_aero(i) * l2_A(i) / (l1_A(i) + l2_A(i)); R2_A_aero(i) = P_A_aero(i) * l1_A(i) / (l1_A(i) + l2_A(i)); R1_R_aero(i) = P_R_aero(i) * l2_R(i) / (l1_R(i) + l2_R(i)); R2_R_aero(i) = P_R_aero(i) * l1_R(i) / (l1_R(i) + l2_R(i)); end %% 🔹 **Verify Load Summation Consistency** total_aero_force = sum(R1_A_aero + R2_A_aero + R1_R_aero + R2_R_aero); total_mass_force = sum(W_ala + W_comb); total_lift_required = n * MTOW / 2; error_check = abs(total_aero_force - total_lift_required); if error_check > 1e-3 warning('❌ Load imbalance detected! Adjustments may be needed.'); else disp('✅ Load distribution verified successfully.'); end %% ✨ **Return Results** results = struct(); results.L = L; results.W_ala = W_ala; results.W_comb = W_comb; results.P_A_aero = P_A_aero; results.P_R_aero = P_R_aero; results.R1_A_aero = R1_A_aero; results.R2_A_aero = R2_A_aero; results.R1_R_aero = R1_R_aero; results.R2_R_aero = R2_R_aero; results.total_lift_required = total_lift_required; results.error_check = error_check; end ". What is your honest take on this?

Your understanding of Stage 3 (main_strength_analysis) is mostly correct, but there are a few refinements to consider based on the tfg_guia_Modelo FEM de la estructura de un ala.pdf and Etapa-3-predimensionado-cajon-ala-metalico-compuesto.pdf.

🔹 Clarification on Stage 3 (Predimensionado & Análisis de Resistencia)

Stage 3 consists of two main parts:

- Predimensionado (Initial Sizing)

  - This step determines initial dimensions for different sections of the wing using force results from Stage 2 (wing and fuselage forces).

  - It involves defining the thickness of panels (revestimiento), spars (largueros), and stringers (larguerillos).

  - This part is not yet a full structural analysis; it's an approximation based on initial material selection.

- Análisis de Resistencia (Structural Strength Analysis)

  - Here, we use forces, materials, dimensions, and the number of stringers per skin to validate structural behavior.

  - This step calculates stresses, stability, and fatigue resistance for the selected sections.

  - Forces need to be correctly assigned to coordinates (ribs, spars, etc.), especially for later FEA (Stage 5).

  - The structural elements must withstand bending (My), shear (V), and torsion (T) forces.

🔹 Key Issues & Solutions

❌ Issue 1: Forces Lack Designated Coordinates

- Forces need to be associated with specific nodes on the structure.

- The calculate_final_forces function computes aerodynamic and mass forces but does not link them to rib intersections.

- Forces should be discretized at costilla-larguero intersections (see Section 3.5.1 of the TFG Guide​).

✅ Solution

- Modify calculate_final_forces.m to output forces at structural points (ribs and spars).

- Store the force-to-structure mapping in TFG_Amora.aviones.(name).forces.

❌ Issue 2: No Clear Structure for Transition from Stage 2 to 3

- Stage 2 computes the forces but does not store them in an easily accessible format.

- The predimensionado part needs to read the forces and derive section properties.

✅ Solution

- Add a function store_forces_in_database.m that:

  - Extracts forces from calculate_final_forces

  - Maps them to rib intersections

  - Saves them into TFG_Amora.aviones.(name).forces

- Modify main_strength_analysis.m to first retrieve these forces.

❌ Issue 3: Variable Wing Box Geometry & Sectional Analysis

- Each section of the wing has different chord lengths, spar positions, and loads.

- Forces must be analyzed per section (at least 5 sections).

✅ Solution

- Ensure that calculate_final_forces outputs forces per rib station.

- Modify predimensionado.m to:

  - Read forces per section

  - Compute initial thicknesses for skin and spars

- Strength analysis should analyze stresses for each section independently.

🔹 My Honest Opinion

Your breakdown of Stage 3 is accurate, but without properly structured force data from Stage 2, the predimensionado step will not be reliable.

I suggest the following order of improvements:

- Modify Stage 2 (Wing & Forces Calculation)

  - Ensure calculate_final_forces outputs forces at structural points.

  - Save this data in TFG_Amora.aviones.(name).forces.

- Update Stage 3: Predimensionado

  - Read forces per section and compute initial thicknesses.

  - Define minimum 5 sections based on chord variation.

- Refine Análisis de Resistencia

  - Use forces, dimensions, materials, and num_larguerillos to compute stresses.

  - Validate results against strength & fatigue limits.

If you want, I can help modify calculate_final_forces and create a store_forces_in_database.m function. Let me know how you’d like to proceed! 🚀