🛠️ Thesis Session Wrap-Up

📊 1. Thesis Session Summary

🔹 Main Objectives We Worked On

✅ Debugging and improving force distribution modeling

- Analyzed the distribution of aerodynamic forces (V_aerodynamic) between the front and rear spars.

- Verified that most of the load is concentrated on the front spar due to the wing’s geometry and sweep.

✅ Ensuring correct force application for FEM simulations

- Evaluated whether the computed loads are physically valid and usable for structural analysis.

- Discussed potential improvements in load distribution between spars to avoid overloading the front spar.

✅ Updating MATLAB functions for multi-load cases

- Implemented three force cases:

  - n=1n = 1n=1 (level flight).

  - n=nlimn = n_{\text{lim}}n=nlim​ (limit load).

  - nult=nlim×1.5n_{\text{ult}} = n_{\text{lim}} \times 1.5nult​=nlim​×1.5 (ultimate load).

- Ensured aerodynamic and mass forces are correctly scaled with nnn.

✅ Improving force visualization in MATLAB

- Used tiledlayout() to compare forces across different load cases.

- Enhanced 3D plotting (stem3, plot3, quiver3) to visualize forces over the 2D wing base.

- Debugged issues where forces were not appearing correctly due to incorrect axis assignments.

✅ MATLAB workspace management improvements

- Used assignin('base', varName, varData) to export key variables from functions to the base workspace.

- Converted two Nx1 vectors into an Nx2 matrix for better data handling in simulations.

🔹 Key Decisions and Conclusions

✅ Aerodynamic load distribution is realistic

- The high front spar loading is expected due to sweep and aerodynamic center placement.

- However, we should validate whether the rear spar is carrying an appropriate share of the load.

✅ FEM-ready forces require additional verification

- Before using these loads in Patran/Nastran, we need to check:

  - Equilibrium of forces → The sum should match n⋅MTOW/2n \cdot MTOW / 2n⋅MTOW/2.

  - Bending moments consistency → The computed moments should match theoretical expectations.

  - Load transfer to rear spar → Ensure it is not underloaded, which could affect torsional rigidity.

✅ Updated force processing workflow

- Created a standardized function to compute aerodynamic and mass forces for different nnn values.

- Improved MATLAB function structure to make force calculations reusable for different cases.

🔹 Challenges Encountered and Solutions Applied

| Challenge | Solution Applied |
| --- | --- |
| Front spar taking nearly all the aerodynamic load | Verified expected behavior due to sweep; suggested validating load transfer to the rear spar. |
| Forces not appearing in 3D plots | Corrected stem3() axis assignments and applied proper scaling. |
| Need to handle multiple load cases efficiently | Used loops and structures to automate force computation for different nnn values. |
| Data not persisting across functions | Used assignin('base', varName, varData) to pass key variables to the workspace. |

🔹 Code Structures and Functions We Implemented/Improved

✅ calculate_forces_for_n.m → Computes forces for different nnn values.
✅ plotAla2D_fuerzas.m → Now supports tiled plots to compare force distributions for different cases.
✅ assignin('base', varName, varData) → Used for exporting important variables for debugging.
✅ Improved data handling → Converted Nx1 vectors to Nx2 matrices for structured force storage.

🔹 Next Steps for the Thesis

📌 Validate force equilibrium and load transfer to the rear spar
📌 Refine force application for FEM simulations (Patran/Nastran export)
📌 Verify moment consistency with theoretical calculations
📌 Run a basic FEM simulation to check spar stress distribution
📌 Document force computation methodology for the thesis write-up

💻 2. Coding Learning Guide Summary

🔹 Core Programming Concepts Applied Today

✅ Coordinate system transformations → Ensuring correct force application on the wing structure.
✅ Interpolation (spline, interp1) → Used for aerodynamic load distribution.
✅ MATLAB struct operations → Organized forces for different nnn cases efficiently.
✅ MATLAB workspace management → Used assignin() to transfer key variables to base workspace.
✅ Plotting in 3D (stem3, plot3, quiver3) → Enhanced force visualization.

🔹 Best Practices in Coding Structure and Function Design

✅ Standardized function inputs/outputs → Allowed easy force calculation for multiple nnn values.
✅ Used loops instead of manually computing forces → Automated the process efficiently.
✅ Debugging with visualizations → 3D plots helped identify load imbalances between spars.
✅ Checked unit consistency → Ensured forces were correctly scaled with nnn.

🔹 Challenges Faced and How We Solved Them

| Coding Challenge | Solution Applied |
| --- | --- |
| Forces not appearing correctly in stem3() | Fixed axis assignments and applied scaling. |
| Needed multiple force cases efficiently | Used loops + structs to compute forces for different nnn values. |
| Debugging variable persistence issues | Used assignin('base', varName, varData) to keep key variables in the workspace. |

🔹 Suggested Exercises & Focus Areas

💡 MATLAB Debugging Practice

- Add assertions (assert()) to verify force sums match expected values.

- Use breakpoints to step through force computations and validate intermediate values.

💡 MATLAB Struct and Data Handling

- Convert forces into structured arrays for easier export to FEM simulations.

- Improve force interpolation accuracy for non-uniform costilla spacing.

💡 FEM Model Validation

- Implement a basic .bdf file writer for exporting forces to Patran/Nastran.

- Run a simple beam analysis to compare against computed loads.

🔹 Recommendations for My Coding Learning GPT

📌 Reinforce coordinate system transformations (global to local reference frames).
📌 Provide debugging workflows for handling interpolation errors (interp1 fails, out-of-range values).
📌 Suggest structured approaches to workspace management (assignin, evalin, persistent variables).
📌 Help automate force export to .bdf format for FEM simulations.

📌 Final Thoughts & Next Steps

🔹 Today’s session helped refine force distribution modeling and set up forces for FEM.
🔹 The next big task is validating these forces before applying them in Patran/Nastran.
🔹 We should also run a simple structural simulation to check stress distribution in the spars.

📌 Would you like me to help debug force equilibrium checks or set up a .bdf export function for your FEM simulation? 🚀