1. Objective and Context

The goal of this stage is to expand our current 2D wing structure model into a 3D representation. This process focuses on the wing section only, as the fuselage geometry can be easily represented due to its simplified structure.

2. Theoretical Basis

- The 3D structure of the wing adheres to a standard NACA airfoil profile as we lack proprietary design data from manufacturers like Airbus​.

- This transformation involves extruding or modifying the z-coordinate across the existing 2D node definitions.

- Following Professor Llamazares' guidelines, the geometry must include vertical surfaces for:

  - Ribs (costillas)

  - Spars (largueros)

These surfaces act as structural partitions across the wing span and are essential for accurate structural analysis​.

3. Methodology

3.1 Coordinate Transformation to 3D

- Existing 2D nodes from datasets such as nodos_larguerillos, nodos_posterior_ala, and nodos_anterior_ala​will be expanded into 3D coordinates.

- A constant z-value will be initially applied across all nodes to simplify the extrusion into 3D space.

- Example MATLAB transformation:

matlab

Copy code

z_value = 0.5; % Placeholder z-coordinate for all wing nodes

nodos_3D = [nodos_2D(:,1), nodos_2D(:,2), z_value * ones(size(nodos_2D,1),1)];

- The above transformation is repeated for each node group:

  - nodos_larguerillos

  - nodos_posterior_ala

  - nodos_anterior_ala

3.2 Rib and Spar Surface Generation

- Ribs (Costillas): Vertical surfaces created between spars and stringers along the spanwise direction.

  - Each rib will connect anterior and posterior spars at discrete sections defined by the distance between ribs.

- Spars (Largueros): Defined as longitudinal structural elements running along the span.

  - Spars are created by extruding node lines along the span.

- Example MATLAB approach for ribs:

matlab

Copy code

for i = 1:numero_costillas

rib_nodes = [nodos_posterior_ala(i,:), nodos_anterior_ala(i,:)];

patch(rib_nodes(:,1), rib_nodes(:,2), rib_nodes(:,3), 'b'); % Visualize rib surface

end

- Visualization: Ensure proper plotting and inspection of vertical ribs and spars using MATLAB's patch and surf functions.

3.3 Temporary ID and Local-to-Global Mapping

- Maintain temporary and global node mappings​​.

- Verify node connectivity across ribs and spars.

- Ensure every node in 3D space has a unique Global Node ID using assignNodeIDs.

4. Structural Integrity and Validation

- Validate that the connections between nodes in the vertical direction (z-axis) align with theoretical expectations from NACA profiles​.

- Check for:

  - Continuity: No gaps between rib and spar surfaces.

  - Symmetry: Ensure proper alignment of nodes along the span.

- Plot and visually inspect results:

matlab

Copy code

figure;

hold on;

scatter3(nodos_3D(:,1), nodos_3D(:,2), nodos_3D(:,3), 50, 'filled');

xlabel('X-axis'); ylabel('Y-axis'); zlabel('Z-axis');

grid on; axis equal;

title('3D Wing Node Representation');

hold off;

5. Next Steps

- Finalize 3D geometry generation for the wing.

- Extend z-coordinate transformations to fuselage nodes.

- Refine node connectivity matrices for vertical and longitudinal surfaces.

- Prepare the geometry for Finite Element Analysis (FEA) mesh generation in Nastran/Patran​.

6. Key Considerations

- Confidentiality: Substitute confidential Airbus data with theoretical NACA profile approximations.

- Scalability: Ensure the methodology allows for more detailed 3D modeling when additional data becomes available.

- Documentation: Maintain clarity in node ID tracking and connectivity matrices for traceability across simulations.

Proposal for Developing a 3D Structural Model of the Wing Section in an Aircraft Finite Element Model (FEM)

1. Introduction

The objective of this proposal is to outline a clear methodology for transitioning our current 2D wing structure model into a 3D representation. This 3D model will focus specifically on the wing section, as the fuselage section can be simplified due to its cylindrical geometry. The 3D representation will adhere to theoretical aerodynamic profiles, specifically NACA profiles, in the absence of proprietary Airbus geometry data.

This transition will enable accurate Finite Element Analysis (FEA) for structural simulations, contributing significantly to the objectives outlined in our Thesis Final Project.

2. Objectives

- Develop a 3D representation of the wing structure based on existing 2D node data.

- Apply a simplified constant z-coordinate transformation for initial geometry generation.

- Generate vertical rib (costillas) and spar (largueros) surfaces to ensure geometric fidelity.

- Validate node continuity and connectivity for the 3D wing structure.

- Prepare the 3D model for FEM simulation using tools like Nastran/Patran.

3. Scope of Work

3.1 Coordinate Transformation to 3D

- Description: Extend the 2D node matrices (nodos_larguerillos, nodos_posterior_ala, nodos_anterior_ala) into a 3D space by introducing a constant z-coordinate across all nodes.

- Methodology:

  - Apply a fixed z-value to all existing 2D nodes.

  - Ensure uniformity in node positioning along the z-axis.

- Expected Outcome:

  - A set of 3D node matrices representing the wing geometry.

MATLAB Example:

matlab

Copy code

z_value = 0.5; % Example z-coordinate

nodos_3D = [nodos_2D(:,1), nodos_2D(:,2), z_value * ones(size(nodos_2D,1),1)];

3.2 Rib and Spar Surface Generation

- Description: Create vertical ribs (costillas) and longitudinal spars (largueros) connecting the anterior and posterior spars, along with stringers.

- Methodology:

  - Generate vertical surfaces between spars and stringers at specified rib intervals.

  - Define longitudinal connections along the wing span.

- Expected Outcome:

  - Defined rib and spar surfaces forming the 3D structural grid of the wing.

MATLAB Example:

matlab

Copy code

for i = 1:numero_costillas

rib_nodes = [nodos_posterior_ala(i,:), nodos_anterior_ala(i,:)];

patch(rib_nodes(:,1), rib_nodes(:,2), rib_nodes(:,3), 'b');

end

3.3 Temporary ID and Local-to-Global Mapping

- Description: Ensure that each node and element in the 3D geometry has a unique Global Node ID for traceability and consistency.

- Methodology:

  - Use the assignNodeIDs function to assign unique IDs to 3D nodes.

  - Map local node IDs to global node IDs using mapLocalToGlobalElements.

- Expected Outcome:

  - Unique identification for all 3D nodes and elements in the model.

3.4 Structural Integrity and Validation

- Description: Validate the continuity, connectivity, and correctness of the 3D geometry.

- Methodology:

  - Verify alignment of ribs and spars.

  - Ensure node connectivity is preserved in the 3D transformation.

  - Visualize using MATLAB's 3D plotting tools (scatter3, patch).

- Expected Outcome:

  - A validated 3D representation of the wing structure ready for simulation.

MATLAB Example:

matlab

Copy code

scatter3(nodos_3D(:,1), nodos_3D(:,2), nodos_3D(:,3), 50, 'filled');

xlabel('X'); ylabel('Y'); zlabel('Z');

grid on; axis equal;

title('3D Wing Node Representation');

4. Tools and Software

- MATLAB: Node transformation, surface creation, and validation plots.

- Nastran/Patran: Finite Element Analysis (FEA) simulations.

- Esacomp: Structural analysis and optimization.

5. Expected Deliverables

- 3D Node Dataset: Including all transformed wing nodes with global IDs.

- Rib and Spar Surfaces: Structured representation of vertical ribs and longitudinal spars.

- Validation Report: Documentation verifying the correctness of the 3D geometry.

- MATLAB Scripts: Code for 3D transformations and plotting.

- Simulation-Ready FEM Model: Exportable dataset for Nastran/Patran analysis.

6. Timeline and Milestones

| Milestone | Task | Estimated Timeframe |
| --- | --- | --- |
| M1 | Coordinate transformation to 3D | 1 week |
| M2 | Rib and spar surface generation | 1 week |
| M3 | Node ID assignment and mapping | 0.5 week |
| M4 | Structural validation and corrections | 0.5 week |
| Total Duration | - | 3 weeks |

7. Key Considerations

- Accuracy: Ensure geometric fidelity with NACA profiles.

- Scalability: Allow easy integration of fuselage geometry.

- Traceability: Maintain clear node and element ID structures.

- Documentation: Provide clear and reproducible MATLAB code.

8. Next Steps

- Finalize initial node transformations into 3D space.

- Create rib and spar surfaces along the wing span.

- Perform validation of the 3D geometry.

- Prepare the FEM model for simulation.

9. Conclusion

This proposal establishes a structured methodology for transitioning from a 2D wing structure to a 3D representation. By adhering to the outlined approach, we ensure clarity, accuracy, and scalability, forming the foundation for robust structural simulations.

Prepared by: Thesis Collaborator
Reviewed by: Jess Bern Amora Ycong

Date: [02-01-2025]