📊 1. Thesis Session Summary

🔹 Main Objectives We Worked On:

✅ Debugging and refining the 3D node generation function (generate_3D_nodes).
✅ Implementing the airfoil thickness (z_extrados, z_intrados) calculation based on NACA 6-series.
✅ Ensuring correct coordinate transformation (localizing x using x_LE).
✅ Reviewing the predimensionado material assignments for cordón, larguerillo, and skins.
✅ Planning the FEM export to Patran/Nastran, defining MAT1, PSHELL, CSHEAR, CROD properties.

🔹 Key Decisions and Conclusions:

✅ Unified material selection → One material for extrados (7075-T6), one for intrados (2024-T3).
✅ Fixed x_local computation → Ensured correct leading-edge reference to avoid out-of-range errors.
✅ Improved interpolation → Used interp1() safely with clamped values to prevent NaN errors.
✅ Confirmed necessary FEM properties → CROD (A_larguerillo, A_cordon), PSHELL (t_si), CSHEAR (t_l).

🔹 Challenges Encountered and Solutions Applied:

| Challenge | Solution Applied |
| --- | --- |
| x_local producing negative values | Clamped x_LE to stay within valid x range of the wing. |
| x_normalized out of airfoil range | Used max(0, min(1, x_local / chord_length)) to ensure safe interpolation. |
| z_extrados = 0 | Verified airfoil thickness data and added debug prints for x_normalized. |
| Material assignments were unclear | Reviewed the guide → Assigned 7075-T6 (extrados), 2024-T3 (intrados). |
| Understanding how to export FEM properties | Identified MAT1 (materials), PSHELL (skins), CSHEAR (spars), CROD (stringers & caps) as required elements. |

🔹 Code Structures and Functions We Implemented/Improved:

✅ generate_3D_nodes → Fixed x_LE, added safe interpolation, and improved logging.
✅ naca6series → Verified thickness data for correct airfoil z-coordinates.
✅ export_to_bdf (planned) → Structured FEM material and element definitions for Patran/Nastran.

🔹 Next Steps for the Thesis:

📌 1. Final Testing & Debugging:
🔹 Validate generate_3D_nodes against realistic airfoil thickness values.
🔹 Check x_normalized values using test cases to ensure correct airfoil lookup.

📌 2. Implement FEM Property Export:
🔹 Define .bdf output functions for MAT1, PSHELL, CROD, CSHEAR elements.
🔹 Create a function to write materials and properties from materialStruct.

📌 3. Validate FEM Model Consistency:
🔹 Check mass, stiffness, and connectivity of the generated structure.

📌 4. Documentation & Thesis Writing:
🔹 Document coordinate transformations (global to local).
🔹 Explain FEM export methodology and material assignments.

💻 2. Coding Learning Guide Summary

🔹 Core Programming Concepts Applied Today:

1️⃣ Coordinate Transformations:
🔹 Used leading edge reference (x_LE) to properly transform global x into local airfoil coordinates.

2️⃣ Interpolation & Data Validation:
🔹 Used interp1() for safe airfoil lookup, ensuring valid range handling.
🔹 Applied max(0, min(1, x_local / chord_length)) to prevent out-of-range errors.

3️⃣ MATLAB Struct and Table Operations:
🔹 Converted tables to struct arrays for FEM material definitions.
🔹 Implemented a struct of arrays vs. struct array approach for cajon.

4️⃣ FEM Modeling & Export Techniques:
🔹 Defined MAT1, PSHELL, CSHEAR, CROD for material and element assignment in Patran/Nastran.

🔹 Best Practices in Coding Structure and Function Design:

✅ Consistent variable naming → Standardized x_LE, x_local, x_normalized for clarity.
✅ Safe value clamping → Ensured x_local ≥ 0, preventing negative airfoil lookups.
✅ Debugging with fprintf → Tracked y_global, x_LE, x_local, x_normalized, z_extrados.
✅ Modular function design → Prepared for FEM export functions (materials, properties, elements).

🔹 Challenges I Faced and How We Solved Them:

| Challenge | Solution Applied |
| --- | --- |
| x_LE values exceeded wing geometry | Clamped x_LE between Lf and max(x). |
| x_local was sometimes negative | Applied max(0, x_local). |
| z_extrados = 0 issue | Verified airfoil thickness data and ensured interp1() was correct. |
| Material assignment confusion | Checked the guide and simplified to 2 materials (7075-T6, 2024-T3). |
| Unclear FEM export steps | Defined necessary MAT1, PSHELL, CSHEAR, CROD properties. |

🔹 Suggestions for Exercises & Focus Areas:

💡 1. MATLAB Debugging Practice:
🔹 Write test cases for generate_3D_nodes with different airfoils and wing configurations.

💡 2. Struct Operations in MATLAB:
🔹 Convert tables to struct arrays and practice FEM material assignments in Patran/Nastran format.

💡 3. FEM Model Validation:
🔹 Implement a basic .bdf file writer for CROD, CSHEAR, and CQUAD4 elements.

💡 4. Coordinate System Transformations:
🔹 Visualize x_LE(y) using plot() to confirm leading edge reference is correct.

🔹 Recommendations for My Coding Learning GPT:

📌 1. Reinforce coordinate transformations (global to local reference frames).
📌 2. Help automate MATLAB struct-to-Patran/Nastran .bdf file generation.
📌 3. Provide debugging workflows for handling interp1() failures and value clamping.
📌 4. Suggest data validation techniques for airfoil thickness calculations.

📌 Final Thoughts & Next Steps

Today, we tackled some key debugging challenges, improved the airfoil transformation logic, and structured the FEM export process for Patran/Nastran.

🔹 Next, we focus on:
✅ Finalizing .bdf export functions
✅ Validating FEM material & element assignments
✅ Testing model consistency

Would you like a test script to validate FEM material assignments automatically? 🚀