In the guide, panels should be defined as quad4. However, there are some surfaces are better suited to be tri. In terms of Nastran calculations, is there a huge difference or it does not really matter?

ChatGPT said:

In Nastran, QUAD4 (four-noded quadrilateral elements) and TRIA3 (three-noded triangular elements) are both commonly used for finite element modeling of surfaces. However, there are some key differences in their behavior and impact on the accuracy of calculations.

1. Accuracy and Numerical Performance

- QUAD4 elements generally provide more accurate results compared to TRIA3 elements for the same mesh density. This is because QUAD4 elements have a better aspect ratio and exhibit lower numerical stiffness.

- TRIA3 elements tend to be overly stiff due to their constant strain approximation, leading to artificial stiffness in bending-dominated problems (like thin panels).

- If you mix TRIA3 and QUAD4 elements, Nastran will handle it, but an excessive number of triangular elements can reduce accuracy.

2. When to Use TRIA3 Elements

- When necessary for meshing: Sometimes, complex geometries or curvature make it difficult to use only QUAD4 elements, and a few TRIA3 elements are unavoidable.

- To avoid very distorted quads: Poorly shaped QUAD4 elements (high aspect ratios, skewness) can be worse than properly shaped TRIA3 elements.

- At transition zones: When refining mesh around features (holes, stiffeners, connections), small triangular elements may be needed.

3. Best Practices

- Prioritize QUAD4 elements whenever possible.

- If TRIAs are necessary, try to keep them below 5-10% of the total elements in the model.

- If the structure is bending-dominated, use quadrilateral elements as much as possible to avoid artificial stiffness.

- If using TRIA3 elements, ensure they are well-shaped (avoid long and narrow triangles).

Conclusion

If your geometry forces you to use some TRIA3 elements, it is not a huge problem, but excessive TRIAs should be avoided to maintain accuracy, especially in bending problems. If you need to use them, try to distribute them evenly and ensure they are well-shaped.

Would you like me to check an example of your mesh distribution?

You said:

When writing for the thesis, I can debate that due to geometries, having tri close to fron spars are better, also they are much smaller, due to the different angles along the root-tip wing. When bending, teoretically speaking, there should not be a lot of "errors" or "artificial stiffiness, because the geometry of our try our elongated along the direction root-tip.

ChatGPT said:

Yes! You can argue in your thesis that due to the complex geometry near the front spar, using triangular elements is more practical because:

- Mesh Quality & Geometry Adaptation

  - Near the front spar, the varying angles of the structure make it difficult to maintain well-shaped quadrilateral elements.

  - Instead of forcing distorted QUAD4 elements, using smaller TRIA3 elements improves local mesh quality.

- Element Orientation & Bending Effects

  - Since your TRIA3 elements are elongated in the root-tip direction, their stiffness effect is less pronounced in the bending direction.

  - The common issue with TRIA3 elements is that they introduce artificial stiffness in bending, but this effect is minimal when they are stretched along the primary load path (root-tip direction).

  - This means your model should not experience significant accuracy loss.

- Boundary Layer Effects Near the Spar

  - The front spar acts as a structural discontinuity, meaning stress gradients are higher in that area.

  - Using smaller TRIA3 elements can help capture the stress variations better compared to large, distorted QUAD4 elements.

How to Justify This in Your Thesis

You can support your argument by:

- Referencing finite element theory (mentioning artificial stiffness and element shape dependency).

- Showing a mesh comparison (distorted QUAD4 vs. small TRIA3).

- Running a convergence study (compare deflections and stress distributions with different meshing strategies).