📚 MATLAB Documentation for Surface Creation in Wing Structural Analysis

This documentation outlines the methodology, reasoning, and workflow for creating surfaces in MATLAB as part of your thesis. The goal is to ensure clarity and provide a reference point for future work, including documentation in your thesis.

✅ 1. Objective

The primary objective of this methodology is to systematically define and create wing surfaces in MATLAB for structural analysis purposes. The surfaces will:

- Exclude Spars Initially: Focus only on stringers and ribs.

- Direction: Follow a rib-to-rib approach (root-to-tip) instead of a stringer-to-stringer approach.

- Focus: Capture structural responses (e.g., moments, loads, shear flows) rather than aerodynamic pressure distributions.

Key Points:

- The directional focus is root-to-tip, instead of stringer-to-stringer.

- Transversal aerodynamic pressure analysis is acknowledged but not the primary focus.

- The surfaces will provide meaningful insights into load distribution and moment changes along the span of the wing.

✅ 2. Motivation for Rib-to-Rib Surface Creation

📐 Why Rib-to-Rib Instead of Stringer-to-Stringer?

- Structural Focus: Moments, loads, and torsional effects vary primarily rib-to-rib along the span.

- Simplified Interpretation: It simplifies the study of how forces propagate from the root (wing box) to the tip (end of the wing).

- Alignment with Structural Elements: Ribs naturally segment the wing structure, providing logical divisions for structural analysis.

🎯 What Is Excluded Initially?

- Spars are initially excluded from the surface creation workflow.

- Aerodynamic pressure field analysis is secondary to structural load propagation.

✅ 3. Workflow Plan

The surface creation process is divided into two main stages:

📊 Stage 1: Horizontal Surfaces Excluding Spars

- Goal: Create initial horizontal surfaces between stringers and ribs.

- Focus: Ensure node connectivity is consistent and accurate.

- Data Sources: Node IDs (id_nodo_local_larguerillo_costilla), rib indices, and stringer indices.

MATLAB Example:

matlab

Copy code

% Example Criteria for Horizontal Surfaces

target_ribs = [5, 6]; % Example: Selected rib indices

target_stringers = [10, 11]; % Example: Selected stringer indices

% Logical Indexing to Filter Nodes

matching_rows = (id_nodo_local_larguerillo_costilla(:,2) == target_ribs(1) & id_nodo_local_larguerillo_costilla(:,3) == target_stringers(1)) | ...

(id_nodo_local_larguerillo_costilla(:,2) == target_ribs(1) & id_nodo_local_larguerillo_costilla(:,3) == target_stringers(2)) | ...

(id_nodo_local_larguerillo_costilla(:,2) == target_ribs(2) & id_nodo_local_larguerillo_costilla(:,3) == target_stringers(1)) | ...

(id_nodo_local_larguerillo_costilla(:,2) == target_ribs(2) & id_nodo_local_larguerillo_costilla(:,3) == target_stringers(2));

% Extract Node IDs

node_ids = id_nodo_local_larguerillo_costilla(matching_rows, 1);

% Define Surface Panel (CCW Order)

P1 = node_ids(1);

P2 = node_ids(2);

P4 = node_ids(4);

P3 = node_ids(3);

% Store Surface Element

surface_panel = [P1, P2, P4, P3];

Validation Plot:

matlab

Copy code

fill([P1(1) P2(1) P4(1) P3(1)], [P1(2) P2(2) P4(2) P3(2)], 'cyan');

xlabel('X-axis'); ylabel('Y-axis');

title('Horizontal Surface Panel');

grid on;

📊 Stage 2: Rib-to-Rib Surfaces

- Goal: Create surfaces that span rib-to-rib in a structured and sequential manner.

- Focus: Analyze load propagation and moment changes along the spanwise direction.

- Data Sources: Node IDs organized by rib indices.

MATLAB Example:

matlab

Copy code

% Example Rib-to-Rib Surface Creation

for rib = 1:(num_ribs - 1)

nodes_rib1 = nodes(nodes(:,2) == rib, :); % Nodes on current rib

nodes_rib2 = nodes(nodes(:,2) == rib+1, :); % Nodes on next rib

% Define Panels (Ensure Correct Ordering)

for i = 1:(size(nodes_rib1, 1) - 1)

P1 = nodes_rib1(i, 1);

P2 = nodes_rib2(i, 1);

P3 = nodes_rib2(i+1, 1);

P4 = nodes_rib1(i+1, 1);

% Store Surface Panel

surface_panel = [P1, P2, P3, P4];

surfaces = [surfaces; surface_panel];

end

end

% Plot Panels for Validation

for i = 1:size(surfaces, 1)

panel_nodes = nodes(surfaces(i, :), :);

fill(panel_nodes(:,1), panel_nodes(:,2), 'cyan', 'FaceAlpha', 0.5);

hold on;

end

hold off;

title('Rib-to-Rib Surfaces');

xlabel('X-axis'); ylabel('Y-axis');

grid on;

✅ 4. Export Workflow from MATLAB to Patran

📤 Export Nodes and Surfaces:

- Nodes: Save node coordinates.

- Surface Connectivity: Define panels with node indices.

matlab

Copy code

% Save Nodes

writematrix(nodes, 'nodes.txt');

% Save Surface Panels

writematrix(surfaces, 'surfaces.txt');

📥 Import into Patran:

- Import Nodes: Use bulk data entry for node coordinates.

- Import Surfaces: Define CQUAD4 or CTRIA3 elements using node indices.

✅ 5. Key Assumptions

- Node Consistency: Nodes are uniquely and consistently indexed.

- Ordering: Counterclockwise order for both quadrilateral and triangular panels.

- Boundary Alignment: No free-floating or unconnected nodes.

- Validation: Plots and numerical checks ensure proper node connectivity.

✅ 6. Expected Results

- Stage 1: Clean horizontal surfaces excluding spars.

- Stage 2: Rib-to-rib surfaces following the structural layout.

- Patran Integration: Seamless import of nodes and surface definitions.

✅ 7. Future Work

- Simulations in Patran: Apply static and dynamic loads to analyze structural responses.

- Load Analysis: Investigate moment, torsion, and shear load propagation.

- Validation: Compare results with analytical or experimental benchmarks.

✅ 8. Final Thoughts

This documentation serves as:

- A reference for your MATLAB scripts.

- A guide for writing your thesis methodology section.

- A foundation for your structural analysis workflow.

If anything needs further clarification, let me know. I’ll ensure this document aligns perfectly with your vision! 🚀😊