📚 Documentation for Thesis and Workflow: Handling Irregular Surfaces Near the Front Spar

✅ 1. Introduction to the Problem

In the structural analysis of wing components, the front spar region often introduces geometric irregularities due to:

- Non-uniform stringer termination points

- Angular deviation of the front spar

- Misaligned node distributions

These irregularities create gaps and uneven surfaces that cannot be accurately modeled using standard quadrilateral panels. The default fallback—using long triangular panels—introduces further complications.

⚠️ Challenges with Long Triangular Panels:

- Poor Aspect Ratios: Long, thin triangles create high aspect ratios, which are numerically unstable in Finite Element Analysis (FEA).

- Unpredictable Load Transfer: Uneven stress and deformation distribution across elongated triangular panels.

- Mesh Instabilities: Poor quality mesh elements can cause simulation convergence issues.

Conclusion: Using long triangular panels near the front spar is not a viable option for accurate structural modeling.

✅ 2. Rationale for Using Smaller Triangular and Quadrilateral Panels

🎯 Why Decompose Long Triangles?

- Improved Mesh Quality: Smaller elements (both triangles and quadrilaterals) maintain better aspect ratios, ensuring numerical stability.

- Accurate Load Transfer: Load paths remain smooth and predictable across smaller elements.

- Stress Resolution: Smaller panels capture localized stress concentrations more effectively.

- Simulation Convergence: Better-conditioned elements reduce the risk of numerical instabilities during simulations.

📐 Design Philosophy:

- Decompose irregular long triangles into a combination of smaller triangles and quadrilateral panels.

- Preserve smooth transitions between triangular and quadrilateral regions.

- Ensure node connectivity remains consistent across adjacent surfaces.

⚖️ Trade-offs:

| Factor | Long Triangles | Small Triangles & Quads |
| --- | --- | --- |
| Aspect Ratio | Poor | Good |
| Stress Accuracy | Low | High |
| Mesh Quality | Poor | Excellent |
| Computational Cost | Lower | Higher |
| Implementation Complexity | Simple | Moderate |

Decision: Prioritize accuracy and stability over computational efficiency.

✅ 3. Implementation Plan

📊 Step 1: Identification of Irregular Zones

- Identify regions where stringers terminate early or where quadrilateral panels cannot be formed.

📊 Step 2: Decompose into Smaller Panels

- Decompose long triangular gaps into:

  - Smaller triangular panels (CTRIA3)

  - Smaller quadrilateral panels (CQUAD4)

📊 Step 3: Separate Data Handling

- Triangular and quadrilateral panels are stored separately in two distinct datasets:

  - quad_surfaces.csv

  - tri_surfaces.csv

📊 Step 4: Validation in MATLAB

- Visualize the panels to ensure:

  - Correct node connectivity

  - No free edges or holes

  - Proper element normals

📊 Step 5: Import into Patran

- Import triangular and quadrilateral datasets separately:

  - CQUAD4: For quadrilateral panels

  - CTRIA3: For triangular panels

- Validate normals, connectivity, and overall mesh quality.

✅ 4. MATLAB Workflow Documentation

📝 MATLAB Code Overview for Decomposition and Separation

matlab

Copy code

% Initialize storage for surfaces

quad_surfaces = [];

tri_surfaces = [];

for index_larguerillo = 1:numero_larguerillos_total

for index_costilla = index_counter_quitar_nodos_larguerillos_menor_Lf(index_larguerillo) + 1 : index_larguerillos_anterior_ala(index_larguerillo) - 1

% Extract nodes for the current surface

matching_rows = (id_nodo_local_larguerillo_costilla(:,2) == index_costilla & id_nodo_local_larguerillo_costilla(:,3) == index_larguerillo) | ...

(id_nodo_local_larguerillo_costilla(:,2) == index_costilla+1 & id_nodo_local_larguerillo_costilla(:,3) == index_larguerillo+1);

node_ids = id_nodo_local_larguerillo_costilla(matching_rows, 1);

% Validate nodes

valid_nodes = node_ids(node_ids ~= -2);

if numel(valid_nodes) == 4

% Quadrilateral Surface

quad_surfaces = [quad_surfaces; valid_nodes'];

elseif numel(valid_nodes) == 3

% Triangular Surface

tri_surfaces = [tri_surfaces; valid_nodes'];

else

disp(['Skipping invalid panel at Rib: ', num2str(index_costilla), ', Stringer: ', num2str(index_larguerillo)]);

end

end

end

% Export the surfaces

writematrix(quad_surfaces, 'quad_surfaces.csv');

writematrix(tri_surfaces, 'tri_surfaces.csv');

📊 Validation Plots in MATLAB

matlab

Copy code

% Plot Quadrilateral Surfaces

for i = 1:size(quad_surfaces, 1)

nodes_quad = nodes(quad_surfaces(i, :), :);

fill(nodes_quad(:,1), nodes_quad(:,2), 'cyan');

hold on;

end

% Plot Triangular Surfaces

for i = 1:size(tri_surfaces, 1)

nodes_tri = nodes(tri_surfaces(i, :), :);

fill(nodes_tri(:,1), nodes_tri(:,2), 'magenta');

end

xlabel('X-axis'); ylabel('Y-axis');

title('Decomposed Front Spar Surfaces');

grid on; axis equal;

hold off;

✅ 5. Expected Results

- Mesh Integrity: No free edges or disconnected nodes.

- Improved Stress Accuracy: Smooth stress and load transitions across mixed elements.

- Numerical Stability: Reduced aspect ratio problems ensure simulation convergence.

- Clear Data Handling: Separate CSV files (quad_surfaces.csv and tri_surfaces.csv) simplify Patran workflow.

✅ 6. Key Thesis Documentation Points

📄 Section Title: "Decomposition of Irregular Surfaces Near the Front Spar for Structural Analysis"

Key Discussion Points:

- Problem Description: Geometric irregularities caused by early stringer terminations.

- Challenges with Long Triangles: Poor aspect ratios, stress artifacts, and numerical issues.

- Solution: Decompose irregular regions into smaller triangular and quadrilateral panels.

- Workflow Steps:

  - Identification of irregular zones

  - Triangular and quadrilateral decomposition

  - Separate storage and export

  - Validation in MATLAB and Patran

- Advantages of the Approach:

  - Accurate stress distribution

  - Mesh integrity

  - Numerical stability

📊 Include Visualizations:

- MATLAB plots showing triangular and quadrilateral surfaces.

- Patran screenshots validating normals and edge continuity.

✅ 7. Final Takeaways

- Focus on Accuracy: Small triangles and quadrilaterals outperform long triangles in irregular zones.

- Data Separation: Clear distinction between triangular and quadrilateral elements in export files.

- Thorough Validation: Both in MATLAB and Patran.

- Documentation: Provide clear diagrams and analysis in the thesis.