I am thinking creating new nodes when creating nodes during the creation of nodes for the stringer. For example, if we are in stringer 10, then we check that we are in the triangular zone, then we include a new node in stringer 9 that is perpendicular to the stringer and matches the node in stringer 9 or 10, because there are some nodes that exist in the previous stringer, but does not in the next stringer and vice versa. Later, on we include the nodes that exist in the front spar to create the surfaces. These surfaces are smaller, a bot of complexity due to the quadrilateral form that has two sides parallel and the other two sides not parallel. There will be more surfaces, but the calculations for the simulation will be better. What do you think of this approach?

3/3

ChatGPT said:

🛠️ Refined Approach to Handle Node Discontinuities in Stringer-Terminated Surfaces

Your approach to address node irregularities by introducing new intermediate nodes in adjacent stringers and aligning them perpendicularly to maintain structural consistency is both innovative and practical. Below, I’ll outline and expand on your idea, highlighting the benefits, potential challenges, and an implementation workflow.

✅ 1. Problem Recap and Understanding

🔍 Key Issues:

- Discontinuities Across Stringers: Nodes present in one stringer may not exist in the next, creating gaps in the mesh structure.

- Triangular Zones: These irregular zones require smaller, tailored panels for accurate finite element analysis.

- Surface Geometry: Panels formed in these zones often result in irregular quadrilateral shapes with two sides parallel and two non-parallel.

🎯 Objective:

- Introduce new intermediate nodes strategically to bridge discontinuities between stringers.

- Ensure the new nodes align perpendicularly to the stringer direction while preserving geometric consistency.

- Adapt the mesh to form smaller panels for better stress distribution and simulation accuracy.

✅ 2. Proposed Workflow

📝 Step 1: Identify Triangular Zones and Discontinuities

- Traverse each stringer and detect where nodes are missing in the subsequent stringer.

- Flag these zones for additional node creation.

Example MATLAB Logic:

matlab

Copy code

% Identify node discontinuities

for index_stringer = 1:num_stringers-1

nodes_current = stringer_nodes{index_stringer};

nodes_next = stringer_nodes{index_stringer+1};

missing_nodes = setdiff(nodes_current, nodes_next);

if ~isempty(missing_nodes)

disp(['Missing nodes in stringer ', num2str(index_stringer+1), ':']);

disp(missing_nodes);

end

end

📝 Step 2: Generate New Nodes in Adjacent Stringers

- For each missing node in a stringer, calculate a perpendicular projection onto the adjacent stringer.

- Ensure new nodes are aligned correctly along the span and perpendicular direction.

Example Node Placement Logic:

matlab

Copy code

% Create perpendicular nodes

for i = 1:length(missing_nodes)

node = missing_nodes(i);

% Extract coordinates (example placeholders)

x_coord = node_positions(node, 1);

y_coord = node_positions(node, 2);

z_coord = node_positions(node, 3);

% Project perpendicularly to stringer line

new_node = [x_coord + offset_x, y_coord, z_coord];

% Add node to adjacent stringer

stringer_nodes{index_stringer+1} = [stringer_nodes{index_stringer+1}; new_node];

end

- Validation: Ensure the new node does not violate existing geometric constraints or create mesh distortion.

📝 Step 3: Adapt Surface Creation Logic

- Use the newly created nodes in conjunction with existing nodes to form smaller quadrilateral panels.

- Handle the triangular zones separately, ensuring node connectivity remains intact.

Example Surface Mapping Logic:

matlab

Copy code

for index_stringer = 1:num_stringers-1

nodes_current = stringer_nodes{index_stringer};

nodes_next = stringer_nodes{index_stringer+1};

for i = 1:length(nodes_current)-1

if ismember(nodes_current(i), nodes_next) && ismember(nodes_current(i+1), nodes_next)

% Create quadrilateral surface

quad_surface = [nodes_current(i), nodes_current(i+1), ...

nodes_next(i+1), nodes_next(i)];

quad_surfaces = [quad_surfaces; quad_surface];

else

% Create triangular surface

tri_surface = [nodes_current(i), nodes_next(i), nodes_next(i+1)];

tri_surfaces = [tri_surfaces; tri_surface];

end

end

end

📝 Step 4: Include Front Spar Nodes for Refinement

- Ensure nodes from the front spar are integrated into the mesh logic.

- Connect these nodes to adjacent stringer nodes to finalize smaller panels.

Logic Example:

matlab

Copy code

% Integrate front spar nodes

for i = 1:length(front_spar_nodes)-1

quad_surface = [front_spar_nodes(i), front_spar_nodes(i+1), ...

stringer_nodes{1}(i+1), stringer_nodes{1}(i)];

quad_surfaces = [quad_surfaces; quad_surface];

end

✅ 3. Advantages of This Approach

- Improved Mesh Quality: Smaller, well-formed panels reduce stress concentration and improve FEM accuracy.

- Adaptability: The logic scales well across varying wing geometries and irregularities.

- Structural Integrity: Preserves the load paths defined by stringer-node relationships.

- Reduced Computational Noise: Smaller panels in irregular regions reduce errors in load transfer predictions.

✅ 4. Challenges and Considerations

- Node Proliferation: Adding nodes increases the computational cost and complexity of the FEM model.

- Perpendicular Alignment: Accurate perpendicular node projection requires geometric precision.

- Validation Overhead: Each adjustment needs validation in both MATLAB and Patran.

✅ 5. Documentation Plan for Thesis

📄 Section Title:

"Adaptive Node Insertion for Addressing Stringer-Terminated Surface Irregularities in Wing Structures"

Key Discussion Points:

- Problem Analysis: Explain node irregularities and their structural implications.

- Intermediate Node Logic: Detail the perpendicular projection strategy for node creation.

- Surface Adaptation: Describe the creation of smaller quadrilateral and triangular panels.

- Validation Workflow: MATLAB visualization → Patran verification → Stress analysis.

📊 Include Figures:

- Diagram showing node discontinuities and newly added nodes.

- Visualization of refined quadrilateral and triangular surfaces.

- Patran validation results demonstrating improved stress distribution.

✅ 6. Final Takeaways

- Node Alignment Matters: Perpendicular projections prevent geometric inconsistencies.

- Smaller Panels Are Better: Enhanced accuracy in FEM results.

- Validation Is Key: MATLAB → Patran → Stress Validation.

- Scalability: This workflow scales across different wing models and irregularities.