Documentation for FEA Aspect Ratio in Horizontal Surfaces

Title: Finite Element Analysis (FEA) Assumptions for Horizontal Surfaces in Wing Structures

Version: 1.0
Author: Aerospace Thesis Collaborator (ChatGPT)
Date: [Insert Current Date]
Reference for Thesis Work: Aerospace Engineering Degree Thesis – ETSIAE, UPM

1. Objective

This document establishes the baseline assumption regarding the aspect ratio of finite element (FE) mesh for horizontal surfaces in the current project phase. It also outlines the plan for potential improvements in future stages.

2. Current Assumption

- Surface Type: Horizontal structural surfaces (e.g., upper and lower wing skins, spars, ribs).

- Element Type: Shell or plate elements, depending on the simulation requirements.

- Aspect Ratio: Rectangular elements with an aspect ratio of approximately 1:2 (length:width).

Justification for Current Assumption:

- Simplifies the initial finite element model.

- Reduces computational complexity for early-stage simulations.

- Provides sufficient accuracy for global structural behavior, particularly in regions with lower stress gradients​​.

3. Known Limitations of the Current Assumption

- Accuracy Concerns: A 1:2 aspect ratio may lead to reduced precision in stress and strain distribution in regions with high stress gradients (e.g., near spars, fasteners, or other discontinuities).

- Numerical Stability: Larger aspect ratios could introduce minor instabilities in results for non-uniform loading conditions.

- Refinement Requirements: Critical load-bearing areas may require finer meshing and closer-to-square aspect ratios for improved accuracy.

4. Future Improvements Plan

- Refinement of Critical Regions: In subsequent stages, critical regions such as spar intersections, root attachment points, and high-stress areas will use elements with an aspect ratio closer to 1:1.

- Advanced Mesh Techniques: Introduction of adaptive meshing strategies to automatically refine regions requiring higher precision.

- Validation Models: Comparison of current assumptions with experimental data or higher-fidelity simulations (e.g., CFD-structural coupling for aerodynamic loads​).

5. Guidelines for Future Development

- Global Analysis (Current Stage): Maintain the 1:2 aspect ratio for broader aerodynamic and structural simulations.

- Local Refinement (Future Stage): Transition to 1:1 aspect ratio in critical zones identified from initial results.

- Documentation Update: Any refinement or deviations must be documented with explanations and validation results.

6. References

- [14†source] Guía básica para realizar el TFG: Modelo FEM de la estructura del ala de una aeronave.

- [15†source] Schrenk Calculation for Aerodynamic Load Estimation.

- [16†source] Predimensionado del cajón del ala metálico-compuesto.

7. Final Note

This document serves as a baseline guideline for our ongoing thesis work. As the project progresses, refinement of the FEA mesh (both globally and locally) will be revisited and adjusted to improve accuracy and simulation efficiency.

End of Document
Please keep this document updated with any significant changes.

Proposal for Improving the Finite Element Model Workflow

Problem Statement:

In our current finite element model (FEM) workflow, rods/elements/lines are created before defining surfaces. This approach introduces significant limitations when refining or adapting the mesh later, particularly when addressing poor aspect ratios (e.g., 1:2) in rectangular elements.

- Key Issues:

  - Difficulty in refining the mesh without disrupting the structural connectivity.

  - Challenges in creating new nodes and redistributing them uniformly across surfaces.

  - Misalignment of nodes when attempting to adapt the mesh for critical areas.

Proposed Solution: Create Surfaces First, Then Define Lines and Rods

Rationale for Change:
Creating surfaces first allows for better control over meshing strategies and enables adaptive refinement of the mesh directly on the surfaces. Lines, rods, and other structural elements can then be generated from surface nodes with consistent alignment and improved accuracy.

Workflow Comparison:

| Current Workflow (Lines First) | Proposed Workflow (Surfaces First) |
| --- | --- |
| Lines/rods are created initially, constraining future surface meshing. | Surfaces are created first, enabling optimal meshing and refinement. |
| Nodes on lines dictate surface discretization, limiting control over element size and aspect ratio. | Mesh density and node placement are determined directly on the surfaces. |
| Adjusting the mesh requires modifying individual lines and reconnecting nodes manually. | Lines and rods are generated from surface nodes, maintaining structural integrity. |
| Complex adjustments required when refining critical regions. | Adaptive meshing becomes feasible in regions requiring refinement. |

Steps for the Proposed Workflow:

- Define Key Surfaces:

  - Create the main surfaces representing the horizontal structures (e.g., wing skins, spars, ribs).

  - Ensure surfaces are continuous and cover the intended structural area.

- Apply Meshing to Surfaces:

  - Generate a mesh directly on the surfaces.

  - Adjust mesh density and aspect ratio as needed (e.g., refine near stress concentration points).

- Extract Nodes from Surfaces:

  - Automatically extract nodes from the meshed surfaces.

- Generate Lines and Rods:

  - Create lines and rods using the extracted nodes.

  - Ensure structural connectivity aligns with the load paths.

- Apply Boundary Conditions and Loads:

  - Define constraints and external forces on both surfaces and structural elements.

Advantages of the Proposed Approach:

- Improved Mesh Quality: Enhanced control over element shapes and aspect ratios.

- Scalability: Easier to refine specific regions without altering the global structural integrity.

- Efficiency: Streamlined process for node and element generation.

- Flexibility: Future adjustments to the geometry or mesh can be handled with minimal disruption.

Potential Challenges:

- Transition Period: Adapting the existing model to follow this new workflow might require initial effort.

- Compatibility: Ensuring compatibility with existing tools and scripts (e.g., Patran/Nastran, MATLAB routines).

Next Steps:

- Test the New Workflow on a Small Section: Apply the surface-first approach to a limited section of the wing structure and compare the results.

- Evaluate the Mesh Quality: Analyze the aspect ratio, node distribution, and overall simulation accuracy.

- Documentation Update: Update modeling guidelines based on findings.

Documentation for FEA Aspect Ratio and Modeling Workflow in Wing Structures

Version: 1.1
Author: Aerospace Thesis Collaborator (ChatGPT)
Date: [Insert Current Date]
Reference for Thesis Work: Aerospace Engineering Degree Thesis – ETSIAE, UPM

1. Objective

This document expands upon the initial aspect ratio documentation (Version 1.0) and introduces a temporary solution for addressing limitations in the finite element modeling (FEM) workflow. It clarifies our short-term and long-term strategies for addressing aspect ratio issues in our wing structure simulations.

2. Current Problem and Context

- The current modeling guidelines suggest a workaround for poor aspect ratios by introducing intermediate ribs​​.

- While this solution helps address localized issues, it does not fundamentally solve the problem of poor aspect ratios arising from creating lines/rods before surfaces.

- Refining the workflow (e.g., switching to a surface-first approach) requires time and effort, which are limited in the current iteration of the thesis.

Current Strategy:

- Proceed with the existing modeling workflow (lines before surfaces, 1:2 aspect ratio).

- Implement intermediate ribs as a workaround in critical areas.

- Document the issue and propose improvements for future iterations.

3. Short-Term Plan (First Iteration)

Goal: Complete the initial modeling and simulation without focusing on enhancing the aspect ratio.

Key Actions:

- Follow Existing Guidelines: Continue using the existing workflow with predefined aspect ratios and intermediate ribs​.

- Minimal Adjustments: Make only critical adjustments to ensure model stability and convergence.

- Document Known Issues: Clearly mention the limitations caused by the current workflow, specifically:

  - Poor aspect ratio (1:2 elements).

  - Limited mesh refinement control.

- Simulation Validation: Ensure the global behavior of the model aligns with theoretical expectations.

Justification for Short-Term Plan:

- Time constraints and the need to produce an initial working simulation model.

- Intermediate ribs provide a functional, albeit suboptimal, workaround.

4. Long-Term Plan (Future Iteration)

Goal: Transition to a surface-first workflow and optimize the aspect ratio across critical regions.

Key Actions:

- Adopt a Surface-First Workflow: Start modeling with surfaces, then derive lines, rods, and ribs from the meshed nodes.

- Improve Mesh Refinement: Use finer meshing in critical regions (e.g., near spars, wing roots, high-stress zones).

- Adaptive Mesh Techniques: Explore automated mesh refinement tools available in Patran/Nastran or MATLAB.

- Validate Refinements: Compare results with the initial model to quantify improvements in stress distribution and convergence.

Justification for Long-Term Plan:

- Improved numerical stability and accuracy.

- Easier to refine high-stress regions without compromising structural continuity.

- Aligns with best practices in FEM for aerospace applications.

5. Integration into the Thesis Document

To maintain clarity in our thesis document, the following structure is recommended:

- Problem Identification: Explain the aspect ratio problem and why it arises from the current workflow.

- Current Solution (Short-Term): Discuss the use of intermediate ribs as a temporary workaround.

- Future Improvements (Long-Term): Propose the surface-first workflow and optimized aspect ratios for future iterations.

- Justification: Clearly state the time constraints and the need to prioritize a functional initial model.

6. Communication and Collaboration Plan

To ensure smooth collaboration and avoid misunderstandings:

- Shared Knowledge Base: Maintain this documentation updated as we refine our approach.

- Regular Check-Ins: Schedule progress reviews to discuss milestones and potential refinements.

- Document Changes: Any significant updates in the modeling workflow should be documented clearly in this file.

- Prioritization: If time allows, prioritize refining critical regions with smaller aspect ratios (1:1).

7. Final Note

This documentation serves as a reference for our modeling strategy during the thesis project. It outlines the reasoning behind our decisions and sets the stage for future improvements.

Key Takeaway:

- Short-Term Focus: Achieve a working FEM model with existing guidelines and intermediate ribs.

- Long-Term Vision: Transition to a more refined workflow (surfaces first) and optimize aspect ratios.

8. Next Steps

- Complete the first iteration of the FEM model following the current guidelines.

- Document key findings and limitations in the thesis draft.

- Identify critical regions where refinements can bring the most value in future iterations.

End of Document
Please keep this document updated with future revisions and insights.

En esta zona, quiero crear una superficie rectangular de los nodos 1461 1462 1535, el problema es que no sé que pongo en el cuarto nodo, porque el larguerillo está en 1535-1536, pero no está alineada al nodo 1462. En ese caso, no pasa nada si la supercie FEA no está completamenta rectangular, pero si es cuadrilateral con unos lados "raros". Otra opción, es poner el nodo 162 que es una supercie "normal" que es igual que los demás. El problema de está opción es que un lado de la superficie no está en el lado del superficie, (1535-1536) no está alineado con (1535-162). Cuál es tu opinion?

ChatGPT said:

Análisis del Problema para Crear la Superficie FEA

Contexto del Problema:

- Quieres crear una superficie cuadrilateral usando los nodos 1461, 1462, 1535 y un cuarto nodo no definido.

- El problema surge porque:

  - El nodo 1536 no está alineado con el nodo 1462, lo que crea una geometría no rectangular.

  - Usar el nodo 162 generaría una superficie más uniforme, pero el lado 1535-1536 no estaría correctamente alineado con 1535-162.

Opción 1: Crear una Superficie Cuadrilateral Irregular (1461-1462-1535-1536)

- Ventajas:

  - Mantiene la continuidad estructural siguiendo la disposición de los nodos existentes.

  - No introduce nodos adicionales ni altera la geometría existente.

- Desventajas:

  - La superficie no será completamente rectangular.

  - Puede generar elementos con un mal aspecto (aspect ratio irregular), lo que podría afectar la precisión de los resultados del análisis FEA.

Aplicación Recomendada:

- Si el área no está sujeta a esfuerzos críticos o si el impacto en la precisión global del análisis es mínimo, esta opción es aceptable en una primera iteración.

Opción 2: Crear una Superficie con el Nodo 162 (1461-1462-1535-162)

- Ventajas:

  - La superficie será más "regular" y coherente con las demás superficies del modelo.

  - Mejor calidad del mallado para los elementos FEA en esa región.

- Desventajas:

  - El lado 1535-1536 no formará parte de la superficie creada, lo que puede romper la coherencia geométrica local.

  - Es posible que necesites ajustar manualmente la distribución de cargas o condiciones de contorno en esta zona.

Aplicación Recomendada:

- Si la precisión local en esta región es importante (e.g., alta concentración de esfuerzos o interacción con otros elementos), esta opción es más robusta.

Opción 3 (Híbrida): Dividir la Superficie en dos Triángulos

- Crear dos superficies triangulares:

  - Superficie 1: 1461-1462-1535

  - Superficie 2: 1462-1535-1536

- Ventajas:

  - Se mantiene la coherencia geométrica entre los nodos.

  - Evita crear una superficie cuadrilateral distorsionada.

  - El mallado triangular se adapta mejor a las geometrías irregulares.

- Desventajas:

  - Los elementos triangulares pueden ser menos eficientes en algunas aplicaciones estructurales.

  - Puede requerir ajustes adicionales en las condiciones de contorno.

Aplicación Recomendada:

- Esta es una solución intermedia válida si las superficies no son extensas ni predominantes en el modelo global.

Recomendación Final:

- Para una primera iteración y si el tiempo es limitado: Usa la Opción 1 (1461-1462-1535-1536).

- Si quieres una mayor calidad en el mallado y precisión local: Usa la Opción 2 (1461-1462-1535-162).

- Si puedes manejar una geometría más compleja y el software lo permite: Usa la Opción 3 (Dividir en dos triángulos).

En la documentación del proyecto, puedes dejar una nota mencionando que esta región será refinada en futuras iteraciones para mejorar el mallado y la precisión local.

¿Qué opción prefieres implementar? 😊