📚 6.10 Proposal: Stringer-to-Stringer Surface Creation for Extrados and Intrados

6.10.1 Objective

The objective is to create extrados (upper surface) and intrados (lower surface) surfaces in 2D using a stringer-to-stringer (str-to-str) approach. This method systematically identifies and connects key points between stringers and ribs to define small surface panels, ensuring clarity, consistency, and smooth transitions across the wing structure.

6.10.2 Understanding the Proposal

✅ Stringer-to-Stringer Methodology Overview

- Starting Point: Begin at the first stringer (str = 1) and first rib (ribs = 1).

- Second Point: Move to the next rib (ribs = ribs + 1) while staying on the same stringer (str = 1).

- Third Point: Move to the next stringer (str = str + 1) while staying on the current rib (ribs = 1).

- Fourth Point: Move diagonally to the next stringer (str = str + 1) and the next rib (ribs = ribs + 1).

These four points will form a surface panel (a quadrilateral).

- Next Panel Start: The next surface starts from ribs = ribs(i-1) + 1 and str = str(i-1).

- Repeat the Process: Continue iterating through stringers and ribs until the entire wing surface is covered.

✅ Visual Representation

scss

Copy code

Rib 1     Rib 2

●---------●       (Step 2: ribs +1, str = 1)

|         |

| Surface |

●---------●       (Step 4: ribs +1, str +1)

↑     ↑

(Step 1)  (Step 3)

- Point 1: (str = 1, ribs = 1) → Starting Point

- Point 2: (str = 1, ribs = ribs + 1) → Next Rib on the same stringer

- Point 3: (str = str + 1, ribs = 1) → Next Stringer on the same rib

- Point 4: (str = str + 1, ribs = ribs + 1) → Diagonal Connection

Each set of four points defines one surface patch.

6.10.3 Advantages of This Approach

✅ Clarity: The panel definition process follows a clear and repeatable logic.
✅ Modularity: Each small surface patch can be validated independently.
✅ Scalability: Easily scalable to any number of ribs or stringers.
✅ Error Reduction: By defining small, manageable surfaces, misalignments can be detected and corrected earlier.
✅ Ease of Transition to 3D: The node connections will remain consistent when extruded into 3D.

6.10.4 Methodology

Step 1: Identify Key Node Groups

- Nodes on Stringers: Extract nodes associated with each stringer (nodos_larguerillos).

- Nodes on Ribs: Identify intersections between ribs and stringers (nodos_posterior_ala, nodos_anterior_ala).

MATLAB Example:

matlab

Copy code

% Example node extraction for stringers and ribs

nodes_stringers = nodos_larguerillos;

nodes_ribs = [nodos_posterior_ala; nodos_anterior_ala];

Step 2: Define Surface Panels

Using the Stringer-to-Stringer Method, loop through stringers and ribs to define surface panels:

MATLAB Example:

matlab

Copy code

% Initialize surface storage

surfaces = [];

% Loop through stringers and ribs

for str = 1:(size(nodes_stringers, 1) - 1)

for ribs = 1:(size(nodes_ribs, 1) - 1)

% Define the four points of the panel

P1 = nodes_stringers(str, ribs);       % Current Stringer, Current Rib

P2 = nodes_stringers(str, ribs+1);     % Current Stringer, Next Rib

P3 = nodes_stringers(str+1, ribs);     % Next Stringer, Current Rib

P4 = nodes_stringers(str+1, ribs+1);   % Next Stringer, Next Rib

% Store surface as a quadrilateral

surfaces = [surfaces; P1; P2; P4; P3];

% Plot the surface for validation

fill([P1(1) P2(1) P4(1) P3(1)], ...

[P1(2) P2(2) P4(2) P3(2)], 'cyan', 'FaceAlpha', 0.5);

hold on;

end

end

xlabel('X-axis');

ylabel('Y-axis');

title('Stringer-to-Stringer Surface Panels');

grid on;

hold off;

Explanation of the Code:

- Iterate over stringers (str) and ribs (ribs).

- Define four points (P1, P2, P3, P4) based on stringer-rib intersections.

- Store these points as a surface panel.

- Plot each surface patch for validation.

Step 3: Validate Surface Panels

- Plot all surface panels together to ensure no gaps or overlapping areas.

- Check directionality (Upper Left → Bottom Right) is consistent across all panels.

Validation Plot:

matlab

Copy code

figure;

hold on;

scatter(nodes_stringers(:,1), nodes_stringers(:,2), 'filled');

xlabel('X-axis'); ylabel('Y-axis');

title('Validation of Stringer-to-Stringer Surfaces');

grid on; axis equal;

hold off;

6.10.5 Outputs

✅ Key Deliverables:

- 2D Surface Panels: Defined using the stringer-to-stringer method.

- Validation Plots: Showing continuous, gap-free surfaces.

- Node Mapping: Clear assignment of nodes for each surface panel.

✅ Documentation Produced:

- 6.4_Stringer_Surface_Definition.md

- 6.5_Stringer_Surface_Validation.md

6.10.6 Challenges and Considerations

- Node Order Consistency: Ensure panels follow a clockwise or counterclockwise node order.

- Continuity Across Panels: No misalignments between adjacent panels.

- Scalability: Ensure the algorithm works for varying numbers of stringers and ribs.

6.10.7 Next Steps

- Finalize 2D surface generation using the stringer-to-stringer approach.

- Validate all panels visually and numerically.

- Proceed to extrude the 2D surfaces into 3D by assigning z-values.

6.10.8 Summary

This approach systematically builds extrados and intrados surfaces panel by panel using a stringer-to-stringer method. The logical node mapping and iterative definition of panels ensure geometric clarity, modular validation, and easy scalability. This structured surface creation forms a solid foundation for transitioning to a 3D model.