📚 Stage 6: Creation of the 3D Structure from 2D Results

6.1 Objective

The goal of Stage 6 is to transition from the 2D structural representation of the wing to a 3D finite element model (FEM). This step integrates previous 2D results into a three-dimensional framework, aligning the structure with theoretical aerodynamic profiles (e.g., NACA airfoils) and ensuring geometric accuracy across all surfaces.

This stage sets the foundation for finite element analysis (FEA), enabling simulations for structural behavior under loads, material analysis, and design optimization.

6.2 Context and Motivation

6.2.1 Why Create a 3D Model?

- The 2D representation captures essential structural characteristics but lacks depth (z-axis) information.

- A 3D model is necessary for accurate load distribution, stress analysis, and aerodynamic simulations.

- Structural inconsistencies in the transition between spars, ribs, and root geometry require corrections that are only visible in a 3D space.

6.2.2 Challenges Addressed in This Stage

- Root Rib Exclusion: Avoid creating surfaces on ribs that originate at the root.

- Rib Surface Alignment: Ensure ribs follow a consistent Upper Left → Bottom Right directionality.

- Geometric Gaps: Resolve large gaps between the front spar and the root corner through new node creation and surface patches.

6.3 Methodology

6.3.1 Step 1: Coordinate Transformation from 2D to 3D

- Objective: Add a constant or interpolated z-value to all nodes from the 2D dataset.

- Approach: Use MATLAB to apply a constant z-axis extrusion across node matrices.

MATLAB Example:

matlab

Copy code

z_value = 0.5; % Placeholder for Z-axis

nodos_posterior_ala_3D = [nodos_posterior_ala, z_value * ones(size(nodos_posterior_ala, 1), 1)];

nodos_anterior_ala_3D = [nodos_anterior_ala, z_value * ones(size(nodos_anterior_ala, 1), 1)];

nodos_larguerillos_3D = [nodos_larguerillos, z_value * ones(size(nodos_larguerillos, 1), 1)];

Expected Output:

- A set of 3D node matrices (nodos_posterior_ala_3D, nodos_anterior_ala_3D, nodos_larguerillos_3D).

6.3.2 Step 2: Rib Surface Creation

- Objective: Generate surfaces for ribs between rear spars and front spars.

- Rule: Skip ribs that start at the root and begin surface creation from ribs that originate at the rear spar.

- Directionality: Ensure ribs are created in an Upper Left → Bottom Right pattern.

MATLAB Example:

matlab

Copy code

valid_ribs = find(rib_start_points == "rear_spar");

for i = valid_ribs

rib_nodes = [nodos_posterior_ala_3D(i,:); nodos_anterior_ala_3D(i,:)];

patch(rib_nodes(:,1), rib_nodes(:,2), rib_nodes(:,3), 'cyan', 'FaceAlpha', 0.5);

end

Expected Output:

- Vertical rib surfaces aligned with the aerodynamic profile of the wing.

6.3.3 Step 3: Geometric Gap Detection and Correction

- Objective: Identify and close gaps between the front spar and the root corner geometry.

- Approach:

  - Detect gap corner coordinates.

  - Create new nodes along the gap.

  - Generate surface patches to close the gap.

MATLAB Example:

matlab

Copy code

gap_node_1 = [x1, y1, z1];

gap_node_2 = [x2, y2, z2];

nodos_gap = [gap_node_1; gap_node_2];

patch([gap_node_1(1) gap_node_2(1)], ...

[gap_node_1(2) gap_node_2(2)], ...

[gap_node_1(3) gap_node_2(3)], 'magenta', 'FaceAlpha', 0.5);

Expected Output:

- New nodes defining the gap surface.

- A patched surface filling the geometric gap.

6.3.4 Step 4: Validation of 3D Geometry

- Objective: Ensure structural integrity and geometric consistency.

- Approach:

  - Plot all nodes and surfaces in 3D.

  - Verify smooth transitions between ribs, spars, and patches.

MATLAB Example:

matlab

Copy code

figure;

hold on;

scatter3(nodos_3D(:,1), nodos_3D(:,2), nodos_3D(:,3), 50, 'filled');

xlabel('X-axis'); ylabel('Y-axis'); zlabel('Z-axis');

grid on; axis equal;

title('Validation of 3D Wing Structure');

hold off;

Expected Output:

- A validated 3D plot showing ribs, spars, and corrected gaps.

6.4 Outputs

✅ Key Deliverables:

- 3D Node Dataset: Node matrices extended into 3D space.

- Valid Rib Surfaces: Surfaces created for ribs originating at the rear spar.

- Gap Surface Patches: Corrected and interpolated surfaces closing geometric gaps.

- Validation Plots: Visualization of the final 3D wing structure.

✅ Documentation Produced:

- 6_3D_Coordinate_Transformation.md

- 6_Rib_Surface_Creation.md

- 6_Gap_Correction.md

- 6_3D_Wing_Validation.md

6.5 Challenges and Considerations

- Root Rib Exclusion: Ensure surfaces are not generated on root ribs.

- Gap Geometry: Accurately interpolate new nodes to prevent structural inconsistencies.

- Validation: Thoroughly check connectivity between ribs, spars, and patches.

6.6 Next Steps

- Finalize node mapping for the 3D wing structure.

- Export node and element datasets for FEM analysis.

- Proceed to Stage 7: Validation and Testing.

6.7 Summary

Stage 6 establishes the 3D structure of the wing, transitioning from the initial 2D representation. This stage resolves geometric issues, ensures alignment of rib surfaces, and prepares the model for finite element simulations.