1. Objective and Context

The purpose of this document is to establish a clear methodology for creating rib surfaces in our 3D wing structure model while addressing geometric inconsistencies. Specifically:

- Exclude root-starting ribs from surface creation.

- Create rib surfaces starting from the rear spar.

- Address and correct large gaps between the front spar and root corner geometry by introducing new nodes and surfaces.

- Ensure rib surfaces follow the defined directional pattern: Upper Left → Bottom Right.

2. Key Observations

2.1 Rib Surface Exclusion Rule

- Root-Starting Ribs: Certain ribs originate at the root of the wing and do not extend along the rear spar.

- Rule: Surface creation will exclude ribs starting at the root and focus on ribs originating from the rear spar.

2.2 Rib Surface Directionality

- Ribs must follow a consistent directional pattern: Upper Left → Bottom Right.

- This pattern ensures geometric alignment and avoids inconsistencies during structural analysis.

2.3 Gap Issue Between Front Spar and Root Corner

- A large geometric gap exists between the front spar and root corner.

- This gap disrupts the continuity of the rib surfaces and structural integrity.

- Solution: Introduce new nodes and surfaces to fill the gap and ensure smooth transitions.

3. Methodology

Step 1: Identify Valid Ribs for Surface Creation

- Analyze all ribs in the structure.

- Identify ribs that start at the rear spar and exclude root-starting ribs.

- Create an index or list of valid ribs for surface creation.

MATLAB Example:

matlab

Copy code

% Identify valid ribs

valid_ribs = find(rib_start_points == "rear_spar");

Step 2: Create Rib Surfaces Along Valid Ribs

- For each valid rib, create a surface using nodes from the rear spar to the front spar.

- Ensure the Upper Left → Bottom Right directional pattern is maintained.

- Use the patch function to visualize and validate each rib surface.

MATLAB Example:

matlab

Copy code

for i = valid_ribs

rib_nodes = [nodos_posterior_ala_3D(i,:); nodos_anterior_ala_3D(i,:)];

patch(rib_nodes(:,1), rib_nodes(:,2), rib_nodes(:,3), 'cyan', 'FaceAlpha', 0.5);

end

- nodos_posterior_ala_3D: Rear spar nodes for the rib.

- nodos_anterior_ala_3D: Front spar nodes for the rib.

Step 3: Identify and Address the Geometric Gap

- Detect the coordinates of the gap corners between the front spar and root corner.

- Calculate the distance across the gap or derive an interpolated surface.

- Create new nodes to fill the gap.

MATLAB Example (Node Creation):

matlab

Copy code

gap_node_1 = [x1, y1, z1]; % Example coordinates for the new node

gap_node_2 = [x2, y2, z2];

nodos_gap = [gap_node_1; gap_node_2];

Step 4: Create a Surface to Close the Gap

- Using the new nodes, create a surface patch to bridge the gap.

- Ensure smooth transitions by interpolating node coordinates if necessary.

MATLAB Example (Surface Patch):

matlab

Copy code

patch([gap_node_1(1) gap_node_2(1)], ...

[gap_node_1(2) gap_node_2(2)], ...

[gap_node_1(3) gap_node_2(3)], 'magenta', 'FaceAlpha', 0.5);

- Validate the surface visually and numerically.

Step 5: Validation and Adjustment

- Plot all rib surfaces and the corrected gap surface.

- Visually inspect for misalignments or discontinuities.

- Adjust node positions or surface definitions if needed.

MATLAB Example (Validation Plot):

matlab

Copy code

figure;

hold on;

% Plot valid ribs

for i = valid_ribs

rib_nodes = [nodos_posterior_ala_3D(i,:); nodos_anterior_ala_3D(i,:)];

patch(rib_nodes(:,1), rib_nodes(:,2), rib_nodes(:,3), 'cyan', 'FaceAlpha', 0.5);

end

% Plot gap correction surface

patch([gap_node_1(1) gap_node_2(1)], ...

[gap_node_1(2) gap_node_2(2)], ...

[gap_node_1(3) gap_node_2(3)], 'magenta', 'FaceAlpha', 0.5);

xlabel('X');

ylabel('Y');

zlabel('Z');

grid on;

axis equal;

title('Validation of Rib Surfaces and Gap Correction');

hold off;

4. Outputs

After completing the steps, the following outputs will be generated:

- Valid Rib Surfaces: Surfaces starting from the rear spar.

- New Gap Nodes: Nodes created to address geometric gaps.

- Gap Correction Surface: Surface bridging the front spar-root gap.

- Validation Plot: 3D visualization of the corrected structure.

5. Key Considerations

- Exclusion Rule: Strictly exclude ribs originating at the root.

- Directional Consistency: Ensure rib direction follows Upper Left → Bottom Right.

- Gap Closure: Prioritize smooth transitions when creating nodes and surfaces in the gap.

- Validation: Use visual inspection and numerical checks to verify structural integrity.

6. Next Steps

- Finalize rib surface creation for all valid ribs.

- Validate and refine gap correction surfaces.

- Prepare the updated 3D structure for FEM meshing.

- Document results and findings for inclusion in the thesis report.

7. Troubleshooting Guide

- Issue: Misaligned rib surfaces.

  - Solution: Double-check node ordering and rib directionality.

- Issue: Gaps remain after surface correction.

  - Solution: Increase node density or interpolate new nodes.