📚 6.8 Documentation: Creation of Extrados and Intrados Surfaces in the 3D Wing Structure

6.8.1 Objective

The goal of this step is to construct the extrados (upper surface) and intrados (lower surface) of the wing structure by assigning specific z-values to the nodes. Following this, the rear spar and front spar nodes will be connected while addressing two key geometric challenges:

- Root-Rear Spar Misalignment

- Gap at the Root-Front Spar Intersection

This step ensures a complete and coherent 3D representation of the wing surface geometry, forming the foundation for subsequent structural analysis.

6.8.2 Context and Challenges

✅ Extrados and Intrados Surfaces

- Extrados: Represents the upper surface of the wing.

  - Defined by setting z=hz = hz=h (where hhh is the thickness or height of the wing profile).

- Intrados: Represents the lower surface of the wing.

  - Defined by setting z=0z = 0z=0.

These two surfaces will define the outer aerodynamic shape of the wing.

✅ Geometric Challenges to Address:

- Root-Rear Spar Misalignment:

  - Certain nodes at the root and rear spar lack proper connectivity due to geometric limitations.

- Gap at Root-Front Spar Intersection:

  - A significant geometric gap exists at the corner where the front spar meets the root.

Both challenges require new node creation and surface adjustments to ensure continuity.

6.8.3 Methodology

Step 1: Define Extrados and Intrados Nodes

Extrados (Upper Surface) Nodes:

- Set z=hz = hz=h for all relevant nodes along the wing spars and stringers.

MATLAB Example:

matlab

Copy code

h = 0.5; % Example height for extrados

nodos_extrados = [nodos_2D(:,1), nodos_2D(:,2), h * ones(size(nodos_2D,1),1)];

Intrados (Lower Surface) Nodes:

- Set z=0z = 0z=0 for all relevant nodes.

MATLAB Example:

matlab

Copy code

nodos_intrados = [nodos_2D(:,1), nodos_2D(:,2), zeros(size(nodos_2D,1),1)];

Expected Output:

- Two sets of 3D node matrices: nodos_extrados and nodos_intrados.

Step 2: Connect Rear Spar Nodes (Address Root-Rear Spar Issue)

- Identify nodes forming the rear spar on both the extrados and intrados surfaces.

- Address misalignments at the root-rear spar interface by:

  - Creating new interpolated nodes if necessary.

  - Connecting nodes with logical connectivity matrices.

MATLAB Example:

matlab

Copy code

% Rear Spar Connection Example

for i = 1:length(rear_spar_nodes)

node_extrados = nodos_extrados(rear_spar_nodes(i), :);

node_intrados = nodos_intrados(rear_spar_nodes(i), :);

patch([node_extrados(1), node_intrados(1)], ...

[node_extrados(2), node_intrados(2)], ...

[node_extrados(3), node_intrados(3)], 'cyan', 'FaceAlpha', 0.5);

end

Expected Output:

- A rear spar surface connecting the extrados and intrados surfaces, with corrections applied to root misalignments.

Step 3: Connect Front Spar Nodes (Address Root-Front Spar Gap)

- Identify nodes forming the front spar on both the extrados and intrados surfaces.

- Address the gap at the root-front spar corner by:

  - Detecting coordinates of the gap edges.

  - Creating new nodes to bridge the gap.

  - Generating surface patches between these nodes.

MATLAB Example:

matlab

Copy code

% Define new nodes to close the front spar gap

gap_node_1 = [x1, y1, h]; % Extrados gap node

gap_node_2 = [x2, y2, 0]; % Intrados gap node

% Create surface patch to close the gap

patch([gap_node_1(1) gap_node_2(1)], ...

[gap_node_1(2) gap_node_2(2)], ...

[gap_node_1(3) gap_node_2(3)], 'magenta', 'FaceAlpha', 0.5);

Expected Output:

- A front spar surface connecting the extrados and intrados surfaces, with a newly created surface bridging the root-front spar gap.

Step 4: Validation Plot

- Validate the surfaces created for extrados, intrados, and connections at the spars.

- Check for geometric continuity and alignment.

MATLAB Example:

matlab

Copy code

figure;

hold on;

% Plot extrados surface nodes

scatter3(nodos_extrados(:,1), nodos_extrados(:,2), nodos_extrados(:,3), 'filled');

% Plot intrados surface nodes

scatter3(nodos_intrados(:,1), nodos_intrados(:,2), nodos_intrados(:,3), 'filled');

% Plot rear and front spar surfaces

plot3(rear_spar_nodes(:,1), rear_spar_nodes(:,2), rear_spar_nodes(:,3), '-o');

plot3(front_spar_nodes(:,1), front_spar_nodes(:,2), front_spar_nodes(:,3), '-o');

xlabel('X-axis');

ylabel('Y-axis');

zlabel('Z-axis');

grid on;

axis equal;

title('Validation of Extrados, Intrados, and Spar Connections');

hold off;

Expected Output:

- A clear 3D visualization showing:

  - Extrados surface.

  - Intrados surface.

  - Rear spar connections.

  - Front spar connections, including gap corrections.

6.8.4 Outputs

✅ Key Deliverables:

- Extrados Nodes Dataset: 3D coordinates with z=hz = hz=h.

- Intrados Nodes Dataset: 3D coordinates with z=0z = 0z=0.

- Rear Spar Surface: Corrected rear spar geometry.

- Front Spar Surface: Corrected front spar geometry.

- Validation Plot: Visualization of the complete extrados and intrados surfaces with corrected spars.

✅ Documentation Produced:

- 6.1_Extrados_Intrados_Definition.md

- 6.2_Rear_Spar_Connection.md

- 6.3_Front_Spar_Gap_Correction.md

- 6.4_Spar_Surface_Validation.md

6.8.5 Challenges and Considerations

- Geometric Accuracy: Ensure smooth transitions between extrados, intrados, and spars.

- Gap Correction: Avoid abrupt surfaces when creating new nodes to bridge gaps.

- Validation: Verify structural continuity in both visual plots and numerical datasets.

6.8.6 Next Steps

- Finalize node mapping for extrados and intrados surfaces.

- Verify spars’ structural connectivity.

- Proceed to Stage 7: Validation and Testing.

6.8.7 Summary

This step creates the upper (extrados) and lower (intrados) surfaces of the wing while addressing geometric misalignments and gaps at the spars. The result is a continuous, validated 3D surface ready for finite element meshing and analysis.