📌 Standardized Documentation for the Design Process Using Our System

🎯 Purpose of This Documentation

This document provides a clear and structured workflow for using our MATLAB-based automated aircraft wing design system.
It serves two key objectives:
1️⃣ Reference Guide for the Thesis → Ensures that all methodology and steps are systematically recorded.
2️⃣ User Guide for Efficient Design Process → Helps any user (including ourselves) understand how to use the system step by step.

📌 Overview of the Design Process Using Our System

The system follows a sequential design process, ensuring a smooth transition from structural definition to validation.
Below is the full pipeline, emphasizing how the user should interact with the system.

🔷 STEP 1: Define Structural Parameters, Aircraft & Materials

👉 User Action: The user must define the initial parameters in datosEstructural, select the aircraft (avión elegido), and specify material properties.

📌 Inputs (Defined in add_datosEstructural.m and material_info.m)

- datosEstructural → Defines structural layout & constraints

- avión → Provides geometric reference (wing shape, span, etc.)

- materials → Defines elastic properties, failure limits, composite or metal selection

✅ What Happens Here?
✔ The system stores all user-defined structural configurations in TFG_Amora.mat.
✔ These parameters will be used to generate the wing structure.

🔷 STEP 2: Generate Wing Structure (generate_wing.m)

👉 User Action: The user runs generate_wing.m to automatically create an initial wing box structure.

📌 Inputs (From datosEstructural & avión elegido)

- Wing geometry (chord distribution, sweep, span)

- Structural layout (spar locations, rib spacing, stringer spacing)

- Material selection

✅ What Happens Here?
✔ The system automatically computes missing design variables, such as:

- Stringer pitch (pitch) → Based on rib spacing & spars

- Number of stringers (n_stringers) → Defined per panel
✔ The initial structure is created & ready for pre-dimensioning.

🔷 STEP 3: Pre-Dimensioning & Analysis (pre_dimensioning.m)

👉 User Action: The user runs pre_dimensioning.m to calculate thickness, reinforcements, and stiffness.

📌 Inputs (From generate_wing.m & force_analysis.m)

- Forces (M_u, V_y, T) → From aerodynamic & load calculations

- Structural geometry (from generate_wing.m)

- Material properties

✅ What Happens Here?
✔ The system computes & assigns:

- Skin thickness (tss, tsi) → From bending moment MuM_uMu​

- Stringer area (A_Ls) → Based on buckling constraints

- Spar dimensions → To carry shear & torsion loads
✔ This creates an optimized structure ready for FEM validation.

🔷 STEP 4: Generate FEM Model (generate_structure.m)

👉 User Action: The user runs generate_structure.m to convert the wing box into a finite element model (FEM).

📌 Inputs (From pre_dimensioning.m)

- Fully defined structure (thicknesses, reinforcements, materials)

- Node & element connectivity

✅ What Happens Here?
✔ The system generates a .bdf (Nastran) file with:

- Nodes & elements

- Material properties & loads

- Boundary conditions
✔ The user can now import the model into Patran for validation.

🔷 STEP 5: Validate (Patran ↔ MATLAB Comparison)

👉 User Action: The user runs simulations in Patran/Nastran, then compares results with MATLAB predictions.

📌 Outputs (Comparison Data)

- Displacements & stresses from FEM

- MATLAB analytical predictions

- Error & correction analysis

✅ What Happens Here?
✔ If FEM results match MATLAB → Proceed to resistance analysis.
✔ If there are discrepancies → Adjust thicknesses, stiffeners, or materials.

🔷 STEP 6: Final Resistance & Structural Checks (analizar_resistencia.m)

👉 User Action: The user runs analizar_resistencia.m to check final structural performance.

📌 Inputs (From FEM & Pre-Dimensioning)

- Stress & strain limits

- Fatigue & buckling safety factors

✅ What Happens Here?
✔ The system evaluates structural safety:

- Yield stress checks

- Buckling factor of safety

- Fatigue life estimation
✔ If all checks pass → Final optimized structure is validated!

🚀 Final Thoughts: How Should a User Approach the Design Process?

A user should think of this system as a structured design workflow, where:
✔ Step 1: Define the structural parameters & aircraft.
✔ Step 2: Generate an initial wing structure.
✔ Step 3: Compute thickness, reinforcements, & stiffness.
✔ Step 4: Convert the structure into a finite element model.
✔ Step 5: Compare FEM results with MATLAB predictions.
✔ Step 6: Perform final resistance & fatigue analysis.

🎯 Next Steps

🔹 Would you like a flowchart or system diagram for easier visualization?
🔹 Should we refine generate_wing.m further before moving deeper into validation?

This ensures the design process is structured, scalable, and easy for future users! 🚀