"I aim to establish a clear and standardized documentation framework to ensure accurate interpretation of the information I provide. This documentation will serve two main purposes: first, it will act as a reference guide for me when writing the thesis later, helping maintain consistency and clarity in the presentation of my ideas. Second, it will facilitate effective communication with you, ensuring that the context and intent of the documented information are fully understood without room for misinterpretation." Our overall steps of what I just described (Datos estructural + avion + materials -> wing -> pre-dimensionado + analysis run -> generate structure + (patran <-> matlab validation) + analysis de resistencia.). Pay closer attention on how the user should use the system to create a design process by using our project.

ChatGPT said:

📌 Standardized Documentation for the Design Process Using Our System

🎯 Purpose of This Documentation

This document provides a clear and structured workflow for using our MATLAB-based automated aircraft wing design system.
It serves two key objectives:
1️⃣ Reference Guide for the Thesis → Ensures that all methodology and steps are systematically recorded.
2️⃣ User Guide for Efficient Design Process → Helps any user (including ourselves) understand how to use the system step by step.

📌 Overview of the Design Process Using Our System

The system follows a sequential design process, ensuring a smooth transition from structural definition to validation.
Below is the full pipeline, emphasizing how the user should interact with the system.

🔷 STEP 1: Define Structural Parameters, Aircraft & Materials

👉 User Action: The user must define the initial parameters in datosEstructural, select the aircraft (avión elegido), and specify material properties.

📌 Inputs (Defined in add_datosEstructural.m and material_info.m)

- datosEstructural → Defines structural layout & constraints

- avión → Provides geometric reference (wing shape, span, etc.)

- materials → Defines elastic properties, failure limits, composite or metal selection

✅ What Happens Here?
✔ The system stores all user-defined structural configurations in TFG_Amora.mat.
✔ These parameters will be used to generate the wing structure.

🔷 STEP 2: Generate Wing Structure (generate_wing.m)

👉 User Action: The user runs generate_wing.m to automatically create an initial wing box structure.

📌 Inputs (From datosEstructural & avión elegido)

- Wing geometry (chord distribution, sweep, span)

- Structural layout (spar locations, rib spacing, stringer spacing)

- Material selection

✅ What Happens Here?
✔ The system automatically computes missing design variables, such as:

- Stringer pitch (pitch) → Based on rib spacing & spars

- Number of stringers (n_stringers) → Defined per panel
✔ The initial structure is created & ready for pre-dimensioning.

🔷 STEP 3: Pre-Dimensioning & Analysis (pre_dimensioning.m)

👉 User Action: The user runs pre_dimensioning.m to calculate thickness, reinforcements, and stiffness.

📌 Inputs (From generate_wing.m & force_analysis.m)

- Forces (M_u, V_y, T) → From aerodynamic & load calculations

- Structural geometry (from generate_wing.m)

- Material properties

✅ What Happens Here?
✔ The system computes & assigns:

- Skin thickness (tss, tsi) → From bending moment MuM_uMu​

- Stringer area (A_Ls) → Based on buckling constraints

- Spar dimensions → To carry shear & torsion loads
✔ This creates an optimized structure ready for FEM validation.

🔷 STEP 4: Generate FEM Model (generate_structure.m)

👉 User Action: The user runs generate_structure.m to convert the wing box into a finite element model (FEM).

📌 Inputs (From pre_dimensioning.m)

- Fully defined structure (thicknesses, reinforcements, materials)

- Node & element connectivity

✅ What Happens Here?
✔ The system generates a .bdf (Nastran) file with:

- Nodes & elements

- Material properties & loads

- Boundary conditions
✔ The user can now import the model into Patran for validation.

🔷 STEP 5: Validate (Patran ↔ MATLAB Comparison)

👉 User Action: The user runs simulations in Patran/Nastran, then compares results with MATLAB predictions.

📌 Outputs (Comparison Data)

- Displacements & stresses from FEM

- MATLAB analytical predictions

- Error & correction analysis

✅ What Happens Here?
✔ If FEM results match MATLAB → Proceed to resistance analysis.
✔ If there are discrepancies → Adjust thicknesses, stiffeners, or materials.

🔷 STEP 6: Final Resistance & Structural Checks (analizar_resistencia.m)

👉 User Action: The user runs analizar_resistencia.m to check final structural performance.

📌 Inputs (From FEM & Pre-Dimensioning)

- Stress & strain limits

- Fatigue & buckling safety factors

✅ What Happens Here?
✔ The system evaluates structural safety:

- Yield stress checks

- Buckling factor of safety

- Fatigue life estimation
✔ If all checks pass → Final optimized structure is validated!

🚀 Final Thoughts: How Should a User Approach the Design Process?

A user should think of this system as a structured design workflow, where:
✔ Step 1: Define the structural parameters & aircraft.
✔ Step 2: Generate an initial wing structure.
✔ Step 3: Compute thickness, reinforcements, & stiffness.
✔ Step 4: Convert the structure into a finite element model.
✔ Step 5: Compare FEM results with MATLAB predictions.
✔ Step 6: Perform final resistance & fatigue analysis.