Should we integrate it to our project? Maybe, in the material section, there is an internal processing for this like lamination process,etc. What is your honest take on this?

ChatGPT said:

✅ Honest Take: Should We Integrate Composite Lamination Processing into Our Project?

Yes, it would be beneficial to integrate composite lamination processing into the materials section of our project. However, there are a few important considerations before deciding how deeply we integrate it.

🔹 Why Should We Integrate Composite Laminations?

1️⃣ Improved Accuracy for Composite Structures

- Right now, our material model assumes homogeneous materials (e.g., aluminum, titanium, or carbon fiber as a single entity).

- But in reality, composite materials behave differently due to anisotropy and require lamination-based calculations.

2️⃣ Direct Impact on Key Calculations

- Thickness Calculation: In metals, we choose a single t value, but in composites, thickness depends on ply stacking.

- Buckling & Stability Analysis: Laminates have different flexural stiffness in different directions.

- Failure Analysis: Metals fail by yielding, but composites require Tsai-Wu, Hashin, or Puck criteria.

3️⃣ Integration into Our Existing MATLAB System

- Instead of treating composites as just another "material", we can process them internally when selected.

- This means our system automatically calculates laminate thickness, stiffness, and failure criteria if the user selects a composite material.

🔹 How Should We Integrate It?

I recommend the following approach to keep the system modular and avoid overcomplicating existing functions.

✅ Step 1: Modify material_info Struct to Include Composite Properties

- Right now, our material_info struct assumes E (Young’s modulus), ν (Poisson’s ratio), and ρ (density).

- Instead, we should allow it to store ply properties (e.g., modulus per ply, thickness per ply, ply angles).

🔧 Updated material_info Struct Example:

matlab

CopyEdit

material_info = struct();

material_info.material_id = 1; % Material ID

material_info.type = 'composite'; % Metal or composite

material_info.ply_thickness = 0.125e-3; % Thickness per ply (meters)

material_info.num_plies = 12; % Total number of plies

material_info.ply_angles = [0, 45, -45, 90, 0, -45, 45, 90, 0, 45, -45, 90]; % Stacking sequence

material_info.E1 = 135e9; % Longitudinal modulus (Pa)

material_info.E2 = 10e9; % Transverse modulus (Pa)

material_info.G12 = 5e9; % Shear modulus (Pa)

material_info.nu12 = 0.3; % Poisson's ratio

✅ Step 2: Modify cajon.m to Process Composite Materials

Instead of treating composites as a single material block, we should:

- Check if the material type is composite.

- Automatically compute laminate stiffness using the ABD matrix.

- Pass the computed thickness to A.Ars, A.Ari, and other calculations.

🔧 Example (Inside cajon.m):

matlab

CopyEdit

if strcmp(material_info.type, 'composite')

% Compute Total Thickness from Ply Count

total_thickness = material_info.num_plies * material_info.ply_thickness;

% Compute ABD Matrix for Stiffness

ABD = compute_ABD(material_info.ply_thickness, material_info.ply_angles, material_info.E1, material_info.E2, material_info.G12, material_info.nu12);

% Override the thickness values

dimensions.tss = total_thickness;

dimensions.tsi = total_thickness;

end

✅ Step 3: Create compute_ABD.m to Compute Laminate Stiffness

This function will:

- Take ply properties.

- Compute the ABD matrix (Classical Laminate Theory).

- Return the stiffness properties needed for structural calculations.

🔧 Example Function:

matlab

CopyEdit

function ABD = compute_ABD(ply_thickness, ply_angles, E1, E2, G12, nu12)

num_plies = length(ply_angles);

h = linspace(-num_plies * ply_thickness / 2, num_plies * ply_thickness / 2, num_plies + 1);

Q = compute_Q(E1, E2, G12, nu12);

ABD = zeros(6,6);

for i = 1:num_plies

Qbar = transform_Q(Q, ply_angles(i));

z1 = h(i);

z2 = h(i+1);

% A Matrix

ABD(1:3, 1:3) = ABD(1:3, 1:3) + Qbar * (z2 - z1);

% B Matrix

ABD(1:3, 4:6) = ABD(1:3, 4:6) + Qbar * (z2^2 - z1^2) / 2;

% D Matrix

ABD(4:6, 4:6) = ABD(4:6, 4:6) + Qbar * (z2^3 - z1^3) / 3;

end

end

🚀 Final Thoughts: Should We Integrate This?

✔ Yes, but we should keep it modular → Composites should be processed inside material_info and cajon.m, not everywhere.
✔ This will make our system more flexible → Users can choose either metal or composite, and MATLAB will handle it.
✔ We avoid overcomplication → Existing functions stay the same, but they pull composite-specific values when needed.

🎯 What’s Next?

🔹 Would you like a fully integrated version of compute_ABD.m and compute_Q.m?
🔹 Or should we first test the modified cajon.m to check if composite properties work?

Let me know how you want to proceed! 🚀